3GPP TSG-SA3 Meeting #112 	S3-233745

Goteborg, Sweden, 14 - 18 August 2023

# Source:	Huawei, HiSilicon

# Title:	New WID on security aspects of Selective SCG Activation

# Document for:	Agreement

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on security aspects of Selective SCG Activation

Acronym: SCGA_Sec

Unique identifier:	xxx

Potential target Release:	Rel-18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x |  |  |
| No | X |  |  | x |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NR_Mob_enh2 | RAN2 | 940098 | Further NR mobility enhancements |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

The selective SCG activation mechanism has been developed by RAN2 group. This mechanism enables  the UE to remain connected to a PSCell while being configured with several conditional reconfigurations, each with a different candidate target PSCell. Later on based on the measurement results associated with these candidate PSCells, the UE can select and execute one of these conditional reconfigurations in order to change PSCell.

In this feature, the MN may provide one or several candidate SCG configuration(s) from one or more candidate SN to the UE. The conditional reconfiguration remains valid after the UE selected a target and executed the target cell access. Furthermore, the UE is allowed to connect to the same SN several times without any further reconfiguration by the network.

However, if the UE switches back and forth to the same PSCell or SN then there is a risk for a key stream re-use based on the current DC security design. Therefore, some additional work is required in SA3 for specifying the security aspects of selective SCG activation based on the agreed solution.

4	Objective

The goal of this work item is to specify the necessary security requirements and procedures for the selective SCG activation feature developed by RAN2.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.501 | New subclause under 6.10 could be added to add the security procedures about Selective SCG Activation | SA#101 (Sept 2023) |  |

6	Work item Rapporteur(s)

Ao Lei, Huawei, leiao@huawei.com

7	Work item leadership

SA3

8	Aspects that involve other WGs

RAN2 will need to refer to the updated SA3 specification for newly defined security procedures.

9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

Ericsson