3GPP TSG-SA3 Meeting SA3#112	S3-233945

# Goteborg, Germany, 14 - 18 Aug 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | draft-CR | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for protection of data exchange in roaming case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China mobile, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3_SEC |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

X.8	Protection of data and analytics exchange in roaming case

X.8.1 General

The protection of data and analytics exchange in roaming case including authorization and anonymization of data/analytics:

-	Authorization at data and analytics level is enforced by the roaming entry NWDAF producer. The parameters used by NWDAF service consumer to request/subscribe to the services provided by NWDAF producer are defined in TS 23.288 [105], clause 6.1.5. Accordingly, the operator authorization policies can be configured locally in the NWDAF producer.  Also, when the NWDAF in one PLMN requests an access token from the NRF in the peer PLMN, the access token request and the access token claims may contain the analytics ID.

-	The roaming entry NWDAF producer is responsible to control the amount of exposed data/analytics and to abstract or hide internal network aspects in the exposed data/analytics. The corresponding mechanisms used to restrict the data/analytics and/or anonymization are subject to the implementation.

.

X.8.b	Procedure for protection of data exchange in roaming case

X.8.b.1 Policies configured locally in Roaming entry NWDAF producer

Figure X.8.b.1-1:  Protection of data exchange when policies configured locally in Roaming entry NWDAF

Pre-requisites:

- Roaming entry NWDAF producer, i.e. NWDAFp shall be pre-configured with a list of allowed data per PLMN.

- If token-based authorization is used, NWDAFc shall have acquired an access token from the PLMN2 to consume the services exposed in Nwdaf_RoamingData_Subscribe/Request APIs.

Step 1: NWDAFc sends Nnwdaf_RoamingData_Subscribe/Request message to NWDAFp to request data.

Step 2: The roaming entry NWDAFp shall verify the service request, including verifying token,the visited PLMN ID and determine whether the requested data are allowed to be exposed to NWDAFc in PLMN1 by pre-configured policies.

Step 3: The roaming entry NWDAFp shall apply the security policies per consumer (PLMN) to the requested data and decide on their anonymization, restriction or desensitization based on operator’s policy.

NOTE: The anonymization, restriction or desensitization mechanisms of analytics are left for implementation.

Step 4: NWDAFp returns the requested and processed data to NWDAFc.

X.8.2.2 Policies configured as extended claims in access token

Figure X.8.b.1-1:  Protection of data exchange when policies configured as extended claims in access token

Pre-requisite:

- hNRF shall be pre-configured with a list of allowed Event ID(s) per PLMNs.

Step 1: NWDAFc request token, the message shall carry the NF consumer type, target NF type, expected NF Service name, home and serving PLMNs and requested Event ID.

Step 2: vNRF forward the token request message to hNRF.

Step 3: The home network hNRF shall verify the token get request, then determine whether the requested analytics implied by the Event ID(s) can be obtained by the visited PLMN according to the pre-configur allowedEvent ID(s) of the visited PLMN.

Step 4: If the verification success, hNRF issue the token,  the allowed Event ID(s) of the visited PLMN shall be included in the token.

Step 5: The vNRF forward the Token_Get Response to NWDAFc

Step 6:  If the requested analytics is within the claim of token, the NWDAFc shall send  Nnwdaf_RoamingData_Subscribe/Request with the issued token to NWDAFp, the allowed Event ID(s) shall be included in the claim of the token.

Step 7: The roaming entry NWDAFp shall verify the service request, including verifying token, PLMN ID and whether the requested data are consistent with the Event ID(s) in the claim of token.

Step 8 and step 9: NWDAF collect data for NF(s).

Step 10: The roaming entry NWDAFp shall apply the security policies per consumer (PLMN) to the requested data and decide on their anonymization, restriction or desensitization based on operator’s policy.

NOTE: The anonymization, restriction or desensitization mechanisms of data / analytics are left for implementation.

Step 11:  Roaming entry NWDAFp returns the requested and processed data to NWDAFc.

*** END CHANGES ***