3GPP TSG-SA3 Meeting #113	S3-234434

# Chicago, US, 06 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security for Selective SCG Activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This draftCR aims to draft the refine the potential details of security of Selective SCG Activation, and is based on the initial text description of draft CR S3-234164. According to S3-233200, SA3 has agreed the following security principles for Selective SCG activation procedure: - The MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC. - The UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. The UE changes the SN Counter upon every SN change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the detailed security mechanism and procedures for selective SCG activation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The security protection for selective SCG activation procedure is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.X (new), Annex A.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.10.2.X	Security mechanism and procedures for Selective SCG Activation

### 6.10.2.X.1		General

In the scenario of selective SCG activation, the MN may provide one or several candidate SCG configuration(s) for one or multiple candidate SN(s) to the UE. The UE may select and execute precisely one of these conditional reconfigurations in order to change PSCell based on the measurement results on candidate target PSCells. The conditional reconfiguration for the selected PScell remains valid after the UE selected the target and executed the target cell access procedure. Thus, the UE is able to connect to the same SN several times without any further reconfiguration by the network.

### 6.10.2.X.2		Security context initialisation for selective SCG activation

In order to prevent key-stream reuse when the UE switches back and forth to the same PSCell or the SN, the MN generates per candadite SN the sequence of multiple distinct SN Counter values during the selective SCG activation procedure. Each SN Counter value is unique, and the sequences are non-overlapping. These SN Counter values per SN shall be provided to the UE by the MN. The UE shall store these values.

For the side of each SN:

After the derivation of sequence of multiple distinct SN Counter values for each candidate SN, the SN Counter values are sent to the UE from the MN. The MN does not need to store the SN Counter values for each SNs after sending them.

Editor’s Note: Further collaborate with RAN2 to decide the specific message for the MN to signal the SN Counter values per SN to UE, and to signal KSN key(s) to SN, respectively.

### 6.10.2.X.3		Security mechanism for UE to access to target SN

When the UE accesses an SCG of a target SN, either for the first time or for a subsequent time based on the same conditional reconfiguration, the UE shall select the first unused SN counter value in order among the stored ones associated with the target SN. Because all values are distinct, selecting the first unused one ensures that it is not previously used together with the current KgNB. The UE then derives the corresponding KSN using the SN Counter as described in Annex A.16.

When the UE accesses an SCG of a target SN, the target SN receives the SN Counter value from the UE. The target SN uses the SN Counter value to request the Ksn derivation from MN.

Editor’s Note: Further collaborate with RAN2 to decide which option (1 or 2 or 3) to use for SN.

# * * * * Start of Second change * * * *

# A.16	Derivation of KSN

This input string is used when the MN and UE derive KSN. The following input parameters shall be used:

-	FC =0x79,

-	P0 = Value of the SN Counter as a non-negative integer,

-	L0 = length of the SN Counter value (i.e. 0x00 0x02).

The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB.

Editor’s note: This Annex A.16 is for Derivation of Ksn for DC, SA3 to analyse and decide whether a new dedicated FC value to be defined for selective SCG.

# * * * * End of changes * * * *