3GPP TSG-SA3 Meeting #113 	S3-234439

# Chicago, US, 06 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0132 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Disclaimer for Indirect Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-24 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Mismatch to the TS 33.501 requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the current version of the TS 33.501 there are three methods for authentication listed and the vendor can select. In the TS 33.117 the test case is considering only one specific test case, ergo, if other method has been selected, then this is leading to a inconsistent NESAS certification result. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NESAS certification is leading to inconsistent outcome. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial draft and presentation |  |  |  |  |  |  |  |  |

******************** Start of 1st Change ************************************************************

##### 4.2.2.2.4	Authentication for Indirect Communication

###### 4.2.2.2.4.1	Correct handling of client credentials assertion validation failure

Disclaimer: The CCA test is representative of the other two methods of authentication as specified in TS 33.501 Clause 13.3.1.2.

Requirement Name: Correct handling of client credentials assertion validation failure

Requirement Reference: TS 33.501 [10], clause 13.3.8.3

Requirement Description:

The verification of the Client credentials assertion is performed by the receiving node, i.e., NRF or NF Service Producer in the following way:

-	It validates the signature of the JWS as described in RFC 7515 [16].

-	If validates the timestamp (iat) and/or the expiration time (exp) as specified in RFC 7519 [17].

If the receiving node is the NR	F, the NRF validates the timestamp (iat) and the expiration time (exp).

If the receiving node is the NF Service Producer, the NF service Producer validates the expiration time and it may validate the timestamp.

-	It checks that the audience claim in the the client credentials assertion matches its own type.

It verifies that the NF instance ID in the client credentials assertion matches the NF instance ID in the public key certificate used for signing the assertion.

Threat References: TR 33.926 [4], clause 6.3.x.1, Incorrect validation of client credentials assertion

Note: The following test case only applies if the NF under test implements verification of client credentials assertions.

Test Case:

Test Name: TC_CLIENT_CREDENTIALS_ASSERTION_VALIDATION

Purpose:

Verify that the NF under test correctly handles client credentials assertion validation failure.

Editor's Note: This test case applies for Rel-16 NFs. The formulation for indicating the applicable release may need to be updated.

Procedure and execution steps:

Pre-Conditions:

-	Test environment with a consumer NF and a SCP, which may be simulated. (Potentially simulated) consumer NF and (potentially simulated) SCP can be combined for the testing purpose.

-	The NF under test is preconfigured with the certificate of the consumer NF.

-	The NF under test is configured to require assertions for NF consumer authentication for at least one of its services.

-	The tester has the private key of the consumer NF.

-	The tester has access to the interface between the consumer NF and the NF under test.

Execution Steps

Test Case 1: Failed verification of the client credentials assertion integrity

1)	The tester computes a client credentials assertion correctly, except that the signature is incorrect, and then includes the client credentials assertion in the service request sent from the consumer NF to the NF under test via the SCP.

2)	The integrity verification of the client credentials assertion by the NF under test fails.

Test Case 2: Incorrect audience claim in the client credentials assertion

1)	The tester computes a client credentials assertion correctly, except that the audience claim is incorrect, i.e., the audience claim in the client credentials assertion does not match the type of the NF under test, and then includes the signed client credentials assertion in the service request sent from the consumer NF to the NF under test via the SCP.

2)	The NF under test verifies that the audience claim in the client credentials assertion does not match its type.

Test Case 3: Expired client credentials assertion

1)	The tester computes an access token correctly, except that the expiration time (exp) has expired against the current time, and then includes the signed client credentials assertion in the service request sent from the consumer NF to the NF under test via the SCP.

2)	The NF under test verifies that the expiration time in the client credentials assertion has expired against the current time.

Expected Results:

For test cases 1~3, the NF under test rejects the consumer NF’s service request and sends back an error message.

Editor's Note: the result needs to be aligned with the relevant error handling description to be added in TS 29.500.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot containing the operational results.

******************** End of 1st Change ************************************************************