3GPP TSG-SA3 Meeting #113 	S3-234475

# Chicago, US, 6 - 10 November 2023

# Source:	3GPP RAN3

# Title:	LS on QMC support in RRC_IDLE and RRC_INACTIVE

# Document for:	Information, Discussion

Agenda Item:  3

3GPP TSG-RAN WG3 Meeting #121	R3-224745

# Toulouse, France, August 21th – 25th 2023

Title:	LS on QMC support in RRC_IDLE and RRC_INACTIVE

Response to:	-

Release:	Rel-18

Work Item:	NR_QoE_enh-Core

Source:	RAN3

To:	RAN2, SA2

Cc:	SA5, SA3

Contact Person:

Name: 			Filip Barac

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description:

To ensure signalling-based and management-based QoE measurement continuity for MBS across RRC states, RAN3 agreed that the QoE measurement configuration information should be made available to the gNB serving the UE when the UE transits from the RRC_IDLE to RRC_CONNECTED state. In that respect, RAN3 is discussing whether the QoE measurement configuration information should be stored at the UE or at the AMF while the UE is in RRC_IDLE state. After the UE transits from the RRC_IDLE to RRC_CONNECTED state, this information should be provided to the serving gNB. The QoE measurement configuration information to be stored includes the following, per QoE configuration:

- QoE reference.

- The IP address or ID of the Measurement Collection Entity.

- The measConfigAppLayerID.

- Service type.

- QoE measurement type (s-based or m-based measurement) for MBS broadcast service.

- (Working Assumption): available RAN visible QoE metrics.

- Additional information to be stored is FFS.

In addition, RAN3 agreed that, after a UE configured for QoE measurements transits from the RRC_IDLE to the RRC_CONNECTED state, the gNB serving of the UE should be aware of the status of MBS QoE measurement session.

2. Actions:

To SA2: RAN3 respectfully asks SA2 to inform RAN3 whether there are any technical issues with storing the above listed QoE measurement configuration information at the AMF. The QoE measurement configuration information stored at the AMF would be transparent to the AMF.

To RAN2: RAN3 respectfully asks RAN2 to inform RAN3 whether there are any technical issues with storing and retrieval of the above listed QoE measurement configuration information at UE, and to discuss how to ensure that the gNB serving the UE upon transition from the RRC_IDLE to the RRC_CONNECTED state, is aware of whether there are ongoing QoE measurements for MBS.

3. Dates of future TSG RAN WG3 meetings:

RAN3#121-bis                       October 9th – 13th, 2023		Xiamen, P.R. China

RAN3#122	November 13th – 17th, 2023		Chicago, IL, USA