3GPP TSG-SA3 Meeting #113	S3-234516

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0016 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to 6.3.7 on discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2023-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 5.2.2 of TS 23.586 [1] contains the following Editor’s Note related to SA3 that is also related to the privacy of sharing the location of a Located UE: Editor's note: Privacy aspects of sharing the location of a Located UE (e.g. to any Target UE) and exposing information about such privacy aspects during discovery (e.g. to facilitate Located UE selection by a Target UE) needs to be aligned with SA WG3. Knowing early on in the procedures if a Located UE is available to a Target UE not only helps Located UE selection, it is also useful to select which Ranging/SL Positioning procedure to request/invoke. For example, if the Located UE is only willing to share its location with the LMF but not with the Target UE, then the Target UE can use this information to select the network-based procedure instead of the network assisted procedure, rather than to find out later that it is not possible for the Target UE to receive the location of the Located UE Whether or not such information can be exposed during discovery can be controlled by the Located UE itself, according to its privacy preferences. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 6.3.7 to also cover the discovery aspect of whether location information is available to the discoverer UE (e.g. during later steps in the Ranging/SL Positioning procedure). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Target UE could not perform proper selection of Ranging/SL Positioning procedure (network-based or network-assisted) and/or proper selection of Located UEs. Also, Editor’s Note in SA2 would not be addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.7	Procedure of UE privacy verification for UE-only operation and discovery

For UE-only Operation in which the network is not involved in Ranging/Sidelink positioning and for discovery, the authorization for UE privacy is based on the local configured privacy verification information to determine whether its location related information can be exposed to the peer UE or not. If during UE-only operation the UE (e.g. Located UE) is requested for its location, the privacy profile allows location exposure, the UE  accepts the request to expose its location related information and proceeds. During discovery, if the privacy profile allows location exposure to the discoverer UE, the discoveree includes a “Location available to discoverer” attribute in the discovery message to the discoverer.