3GPP TSG-SA3 Meeting #113 	S3-234589

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0188 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A-KID update after UPU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_ph2 |  |  |  |  |  | Date: |  |  | 2023-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to 3GPP TS 33.501, the UDM can perform UE Parameters Update (UPU) procedure to update UE’s routing indicator (RID). Then the UE will generate a new A-KID according to the updated RID and use the new A-KID in the AKMA session with the AF. However, the A-KID in AAnF and AF is generated by the old RID if UDM doesn’t trigger the home network triggered primary authentication after performing UPU. The inconsistency of A-KID will lead to a failure in AKMA service. Therefore, mechanism is needed to remove the old AKMA context in the AAnF and update the A-KID in AAnF and AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedure is added to update the A-KID in AAnF and AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AKMA service failure because of the inconsistency of A-KID between UE and AAnF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGE ****

6.X	A-KID update after UPU

New A-KID is used to initiate AKMA service in the UE side due to UE Parameters Update (UPU) procedure. This procedure is used to update the A-KID in the AAnF side.

- Figure 6.X: A-KID update procedure after UPU

- 1. The UDM performs UPU and updates the RID of UE.

- 2. Then the UDM directly triggered the home network triggered primary authentication procedure.

- 3. After primary authentication, AAnF can achieve the new A-KID.

- 4. The AAnF derives the AKMA Application Key (KAF) from KAKMA。

- 5. The AAnF sends Naanf_AKMA_ApplicationKey_Get response to the AF with SUPI, KAF and the KAF expiration time. Whether to send SUPI or GPSI is determined by AAnF based on the local policy.

6. Then the establishment of application session between UE and AF is completed.

**** END OF CHANGE ****