3GPP TSG-SA3 Meeting #113 	S3-234711

# Chicago, US, 6 -10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | 0138 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 ProSe – Adding security for U2U Relay communication with integrated discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The 5G ProSe UE-to-UE Relay Communication with integrated discovery is supported according to TS 23.304. However, the security for 5G ProSe UE-to-UE Relay Communication with integrated discovery is not defined in TS 33.503. Therefore, it is proposed to define the security for 5G ProSe UE-to-UE Relay Communication with integrated discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The security procedure in clause 6.2 of TS 33.503 is used for both PC5 link security between the 5G ProSe End UE and the 5G ProSe UE-to-UE Relay, and the End-to-End link security between the 5G ProSe End UEs via the 5G ProSe Layer-2 UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of security for 5G ProSe UE-to-UE Relay Communication with integrated Discovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF 1st CHANGE*******

### 6.6.X	Security for 5G ProSe UE-to-UE Relay Communication with integrated Discovery

The security procedure in clause 6.2 of present document is used to establish a secure PC5 link between the 5G ProSe End UE and the 5G ProSe UE-to-UE Relay.

The security procedure in clause 6.2 of present document is used to establish End-to-End security link between the 5G ProSe End UEs via the 5G ProSe Layer-2 UE-to-UE Relay.

************ END OF 1st CHANGE********