3GPP TSG-SA3 Meeting # 112	S3-234766

# Chicago, US, 6 -10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1856 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authentication result removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2023-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It’s proposed to add procedures to specify the authentication result removal to address the potential fraudulent issue caused by invalid authentication result. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding two services in AUSF and UDM to perform the authentication result removal procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid authentication result stored in UDM |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause (6.1.4.x, 14.1.x, 14.2.x) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes****

6.1.4.X	Authentication result removal

In the case that the Purge of subscriber data in AMF after the UE deregisters from the network or the NAS SMC fails following the successful authentication the registration procedure, the AMF requests the AUSF to inform the UDM to remove the authentication result. The AMF shall invoke the Nausf_ UEAuthenticationResultRemoval service by sending a Nausf_ UEAuthenticationResultRemoval Request message to the AUSF. The Nausf_ UEAuthenticationResultRemoval Request message shall contain SUPI and serving network name.

The AUSF shall send a Nudm_ UEAuthenticationResultRemoval Request to the UDM for removing the authentication result of the UE after receiving the above message.

Upon reception of the Nudm_ UEAuthenticationResultRemoval Request, the UDM shall mark the status of the authentication as invalid.

**** Next Changes****

14.1.X	Nausf_UEAuthenticationResultRemoval service operation

Service operation name: Nausf_ UEAuthenticationResultRemoval

Description: Remove the authentication result in UDM in the case that the Purge of subscriber data in AMF after the UE deregisters from the network or the NAS SMC fails following the successful authentication the registration procedure.

Input, Required: SUPI, serving network name

Input, Optional: None

Output, Required: None

Output, Optional: None

**** Next Changes****

14.2.X	Nudm_UEAuthenticationResultRemoval service operation

Service operation name: Nudm_UEAuthenticationResultRemoval

Description: Remove the authentication result in UDM in the case that the Purge of subscriber data in AMF after the UE deregisters from the network or the NAS SMC fails following the successful authentication the registration procedure.

Input, Required: SUPI, serving network name

Input, Optional: None

Output, Required: None

Output, Optional: None

**** End of Changes****