3GPP TSG-SA3 Meeting #113 	S3-234796

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.328 | CR | 0072 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update UE terminating procedures for e2DCe |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_SEC |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The figure 7.3.4-1 and the related description text shows that the SDP Offer/Answer goes directly from the S-CSCF to the P-CSCF(IMS ALG) while the flow should have been from the S-CSCF to the IMS AS in a corresponding way as the IMS originating procedure in 7.2.4. The DCMF is renamed in TS 23.228 to MF (Media Function) while in 7.3.4 the term DCMF is still used. The description of the steps is not following the drafting rules with respect to the numbered lists. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Figure 7.3.4-1 and related description text. The DMCF term is replaced with the MF term. Editorial correction for compliance with the 3GPP drafitng rules. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurate specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGE ****

### 7.3.4	UE terminating procedures for e2DCe

Figure 7.3.4-1 shows the terminating session set-up procedures for one or more media streams using e2DCe security.

NOTE 1:	The procedures shown in the figure apply to users located in their home service area. The same concepts apply to roaming users.

Figure 7.3.4-1: Terminating call flow for e2DCe using MF/MRF case

The IMS UE performs an IMS terminating session set-up according to 3GPP TS 23.228 [3], with modifications as described in the following. If both IMS UE and network indicated support for e2DCe-security for IMS data channel media streams during registration and the P-CSCF receives an SDP Offer for an IMS data channel media stream using "UDP/DTLS/SCTP" transport from the S-CSCF, then the P-CSCF (IMS-ALG) shall establish e2DCe-security for the IMS data channel media stream as described in this clause.

NOTE 2: 	The P-CSCF (IMS-ALG) will not establish e2DCe security for IMS data channel media if the SDP offer received from the S-CSCF indicates that e2e security is being offered, cf. clauses 7.3.2, 7.3.3, and 7.3.5 for the establishment of e2e security on the terminating side.

The procedure in the above figure is now described step-by-step.

1.	The S-CSCF in the terminating network receives an SDP Offer for an IMS data channel stream with transport "UDP/DTLS/SCTP" from the originating network.

2.	The S-CSCF performs the required procedures according to TS 23.228 [3] and forwards the SDP Offer for the media stream to the IMS AS.

3.	The IMS-AS checks the media streams in the SDP Offer.

For each IMS data channel media stream offered with transport "UDP/DTLS/SCTP",

-	if both the IMS UE and P-CSCF (IMS-ALG) indicated support for e2DCe-security for IMS data channel during registration, the IMS-AS and S-CSCF proceed for the respective media stream as described in this clause and allocates the required resources, includes the Media Function/MRF in the media path for establishing the DTLS towards the IMS UE and retrieves from the Media Function/MRF the fingerprint, tls-id, and setup role of the certificate the Media Function/MRF is going to use for setting up security for this media stream.

-	if not both the IMS UE and P-CSCF (IMS-ALG) indicated support for e2DCe-security for IMS data channel during registration, the IMS-AS should continue using e2e security, as described in clause 7.3.5 of the present specification.

NOTE 3: 	The inclusion of the Media Function/MRF in the media path is required for the purposes of e2DCe security even if it was not required otherwise.

The IMS-AS modifies the SDP Offer before sending the SDP Offer back to S-CSCF.

For e2DCe protection of an IMS data channel media stream, the IMS-AS keeps the transport as "UDP/DTLS/SCTP" in the SDP Offer, conveys the fingerprint, tls-id, and setup attribute information of the MF/MRF in accordance with RFC 8841 [48] as well as an indication that e2DCe security is offered by the network to UE.

The IMS-AS then sends the updated SDP Offer to S-CSCF.

4.	The S-CSCF performs the required procedures according to TS 23.228 [3] and forwards the SDP Offer for the media stream to the P-CSCF(IMS-ALG).

5.	The P-CSCF (IMS-ALG) forwards the SDP Offer to the IMS UE B.

6.	IMS UE B replies with an SDP Answer for a secured media stream.

For e2DCe protection of IMS Data Channel, the IMS UE B includes in the SDP Answer the fingerprint of the UE´s certificate, tls-id, and setup attributes in accordance with RFC 8841 [48].

7.	The P-CSCF sends the SDP Answer to the S-CSCF.

8.	The S-CSCF performs the required procedures according to TS 23.228 [3] and sends the SDP Answer to the IMS AS.

9.	The IMS-AS communicates the cryptographic information contained in the SDP Answer to the Media Function/MRF, performs the required procedures according to TS 23.228 [3] and sends the SDP Answer to the S-CSCF.

For e2DCe protection of IMS Data Channel, the cryptographic information communicated to the Media Function/MRF consists of the fingerprint of the IMS UE B’s certificate, tls-id, and setup attributes in accordance with RFC 8841 [48]. The S-CSCF instructs the Media Function/MRF to verify during the subsequent DTLS handshake with the IMS UE (see step 11) that the fingerprint of the certificate passed by the IMS UE during this DTLS handshake matches the fingerprint passed by the S-CSCF to the Media Function/MRF.

10.	The S-CSCF forwards the SDP Answer towards the originating network.

11.	In case of IMS Data Channel, when the full session setup has been completed, the DTLS connection shall be established between the IMS UE and the Data Channel Media Function/MRF. When subsequently media are sent from or to the IMS UE, the Data Channel Media Function/MRF performs the required DTLS specific cryptographic operations on the media.

NOTE 4:	 It is left to stage 3 specifications whether the IMS UE takes the role of DTLS client or DTLS server. These alternatives are equivalent from a security point of view.

**** END OF CHANGE ****