3GPP TSG-SA3 Meeting #113 	S3-234885

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0021 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to AF authorization procedure for Ranging/SL positioning service exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For authorization of AF/5GC NF for Ranging/SL positioning service exposure, it is currently specified that “If all of the UEs don't grant permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request from the AF/5GC NF”. It implies that the GMLC shall reject the service request only when none of the n UE grants permission for Ranging/SL positioning exposure and the GMLC shall accept the service request when one or more UE grant permission. This is logically incorrect because: - In case only one of the n UE grants permission and n-1 UEs do not grant permission, the GMLC shall actually reject the service request also, because Ranging/SL positioning service concerns at least two UEs. - Even if only one of the n UE does not grant permission and n-1 UEs grant permission, the GMLC may also need to reject the service request when the UE not granting permission is the Target UE to be measured. - The n UEs are either designated by the application or selected by the network (e.g. LMF). If not all the designated/selected UEs can participate in the Ranging/SL positioning service due to privacy concerns (part of the UEs allows, part of the UEs disallows), how the service shall proceed is up to the application or the network operator. If the GMLC accepts and forwards the service request by excluding the UE(s) not granting permission from the service, the actual number of UEs involved in Ranging/SL positioning operation is less than the designated/selected number. Then the final ranging result may not meet the requirement of the service requestor, e.g. the ranging accuracy can be obtained only by using the full number of n UEs. Therefore, it is not appropriate to mandate that the GMLC shall reject the service request when all the UEs don’t grant permission. It is proposed that how the GMLC determines to accept or reject the service should be based on a local rule, which can be configured by the operator or sent from the application. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the changes that the GMLC shall decide to proceed with or reject the service request from the AF/5GC NF based on a local rule, which may be configured by the network operator or sent by the requesting AF/5GC NF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inapproriate handling of the service request from the AF/5GC NF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

6.3.5	Procedures for authorization of AF/5GC NF for Ranging/SL positioning service exposure

For the authorization of the AF or 5GC NF for Ranging/SL Positioning service exposure, the SL-MT-LR procedure specified in TS 23.273 [3] is taken as the baseline. The authorization shall be performed towards all the n UEs (n ≥ 2), i.e. UE1, UE2, ..., UEn in the request message. If all of the UEs grant permission for Ranging/SL Positioning exposure, the GMLC shall forward the service request from the AF/5GC NF to the AMF. If none of the UEs grants permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request. If part of the UEs grant and part of the UEs don't grant permission for Ranging/SL Positioning exposure, the GMLC shall decide to proceed with or reject the service request from the AF/5GC NF based on a local rule, which may be configured by the network operator or sent by the requesting AF/5GC NF.

When receiving the Ranging/SL Positioning service request from the AF/5GC NF, the GMLC interacts with the UDM to check the UE privacy profile. The UE LCS Privacy Profile defined in clause 5.4.2 of TS 23.273 [3] is taken as the baseline for the UE privacy profile for Ranging/SL positioning services.

NOTE:	The details of the UE privacy profile for Ranging/SL positioning services needs to be align with SA2.

The GMLC interacts with the AMF to request the ranging result of UEs, which may include an indication of a privacy related action. If the indicator of privacy check related action indicates that the UE must either be notified or notified with privacy verification, a notification invoke message is sent to the UE if the signalling connection established. However, if the Ranging/SL Positioning service is disallowed by the UE, or signalling connection establishment fails and UE notification (including UE notification with privacy verification) is required, the AMF shall provide failure response to the GMLC. If part of the UEs allow and part of the UEs disallow Ranging/SL Positioning exposure, the GMLC shall decide to proceed with or reject the service request based on a local rule, which may be configured by the network operator or sent by the requesting AF/5GC NF.

*************** End of the Change ****************