3GPP TSG-SA3 Meeting #113 	S3-235000

# Chicago, US, 6 - 10 November 2023													rev of S3-234971

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 43.020 | CR | 0083 | rev | 1 | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Prohibition of GEA2 due to security concerns |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI6 |  |  |  |  |  | Date: |  |  | 2023-11-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Several issues relating to GEA1 and GEA2 were tackled in earlier meetings, e.g. in CR0059r1 to TS 43.020 (in S3-212362/SP-210488). However, recent communication from GCF in S3-234488 and ongoing industry developments show that these CRs were insufficient. In 2023, GEA2 does not provide sufficient security strength to prevent real-time cryptographic attacks on 3GPP networks and handsets. Therefore requiring mandatory support for GEA2 in any 3GPP specification used in current market products is a potential critical security vulnerability and therefore all support for GEA2 needs to be prohibited in all future devices in any release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause D.4.9 to remove all support for GEA2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mandated support for a broken algorithm is currently required by 3GPP specifications. This represents a significant security weakness. By not approving this CR, specifications continue to require manadatory support for a known serious security vulnerability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## D.4.9	Support of GPRS-A5 Algorithms in MS

It is mandatory for GEA3 and non-encrypted mode (i.e. GEA0) to be implemented in mobile stations. GEA4 may be implemented in the mobile stations. In particular, GEA1 and GEA2 are prohibited and shall not be implemented in mobile stations.

No other GPRS encryption algorithms shall be supported in mobile stations.