3GPP TSG-SA3 Meeting #113 	S3-234412

# Chicago, United States, 6th Nov 2023 - 10th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0129 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SCAS Definitions and abbreviations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile US, Deutsche Telekom, ZTE Corporation, BSI, Nokia, Ericson, Huawei, Telus, MITRE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current TS uses many undefined terms regularly but they are not referenced/defined. Additionally, ther are numerous abbreviations used that are not listed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update undefined terms, missing definitions, missing abbreviations and references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non 3GPP parties (e.g. labs) will not understand the meaning of frequently used terms and abbreviations that are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 4.4.1, 4.1.1, 4.2.3.2.2, 4.2.3.2.4, 4.2.3.4.1.2, 4.2.3.4.6.2, 4.2.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of 1st Change ****************

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Developer: A creator of systems, components, or services for use on or with a 3GPP network.

Expert knowledge: Possesing skills, training and experience in analysing and understanding security threats in a wide variety of situations.

Local access: Communication through a direct network access interface.

Machine Accounts: These will be used for authentication and authorization from system to system or between applications on a system and cannot be assigned to a single person or a group of persons.

Personal data: any information relating to an identified or identifiable natural person ('data subject').

Identifiable person: one who can be identified, directly or indirectly, in particular by reference to an identification number, name or to one or more factors specific to his physical, physiological, mental, economic, cultural or social identity.

NOTE: 	personal data can be gathered from user data and traffic data.

Network Element: As defined in TS 23.501[18]

Network Function: As defined in TS 23.501[18]

Network product: As defined in TR 33.916[19]

Network product class: As defined in TR 33.916[19]

Operator: The entity which offers telecommunications services over an air interface.

Owner: The person or organization responsible for creating and maintaining content. The person or organization determines who has access to the content and the content permissions.

Producer: A company that sells 3GPP network software or hardware under their own brand.

Remote access: Communication through an external network access interface.

Screenshot: A digital image that shows the contents of a display.

Sensitive data: data that may be used for authentication or may help to identify the user, such as user names, passwords, PINs, cryptographic keys, IMSIs, IMEIs, MSISDNs, or IP addresses of the UE, as well as files of a system that are needed for the functionality such as firmware images, patches, drivers or kernel modules.

System Function: A set of NFs providing services to other authorized NFs.

System group account: a predefined system account in the network product, usually with special privileges, which has a predefined user id and hence cannot be tied to a single user (individual) in a normal operating environment.

EXAMPLE:	the 'root' account.

Vendor: A commercial supplier of 3GPP network software or hardware.

Vulnerability: As defined in TR 33.916[19]

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAA	Authentication Authorization and Accounting

BOOTP	Bootstrap Protocol

BVT	Basic Vulnerability Testing

CAPTCHA	Completely Automated Public Turing test to tell Computers and Humans Apart

CD		Compact Disk

CDP	Customer Data Platform

CIS	Center for Internet Security

COTS	Commercial Off The Shelf

CVE	Common Vulnerabilities and Exposures

DTLS	Datagram Transport Layer Security

DVD	Digital Video Disk

FOSS	Free Open-Source Software

FTP	File Transfer Protocol

GTP-C	GPRS Tunnelling Protocol for Control Plane

GUI	Graphical User Interface

GUID	Globally Unique Identifer

IKE	Internet Key Exchange

IPsec	Internet Protocol Security

JSON	Java Script Object Notation

JWS	JSON Web Signature

LLDP	Local Loop Demarcation Protocol

MOP	Maintenance Operations Protocol

NRF	Network Repository Function

NSI	Network Slice Instance

NSI ID	Network Slice Instance Identifier

NSSAI	Network Slice Selection Assistance Information

OAM	Operations, Administration and Maintenance

PHP	Hypertext Preprocessor

PM	Performance Management

QMS	Quality Management System

RAM	Random Access Memory

RBAC	Role Based Access Control

RCP 	Remote Copy Program

RDP	Remote Delete Program

RPH	Reverse Path Filter

RSH	Remote shell

SBA	Service Based Architecture

SBI	Service Based Interfaces

SCAS		Security Assurance Specification

SCP	Service Communication Proxy

SECAM	Security Assurance Methodology

SEPP	Security Edge Protection Proxy

SFTP	Secure File Transfer Protocol

SGID	Special Group Identification

S-NSSAI	Single Network Slice Selection Assistance Information

SSH 	Secure Shell

SSI	Server Side Includes

SSO	Single Sign-On

SUID	Set owner User Idenification

SYN	Synchrounous Transmission

TFTP	Trivial File Transfer Protocol

ToE	Target of Evaluation

UID	Unique Identifier

WAS	Web Application Security

WebDAV	Web Distributed Authoring and Versioning

*************** End of 2nd Change ****************

*************** Start of 3rd Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 41.001: "GSM Specification set".

[3]	IETF RFC 3871: "Operational Security Requirements for Large Internet Service Provider (ISP) IP Network Infrastructure".

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	CVE-1999-0511, http://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-1999-0511

[6] 	"Practical recommendations for securing Internet-connected Windows NT Systems", https://support2.microsoft.com/default.aspx?scid=kb;%5BLN%5D;164882.

[7]	X-Force Vulnerability Report, http://www.iss.net/security_center/static/193.php

[8]	IETF RFC 2644: "Changing the Default for Directed Broadcasts in Routers."

[9]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[10]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 6749: "OAuth2.0 Authorization Framework".

[13]	3GPP TS 29.501: "Principles and Guidelines for Services Definition".

[14]	Void

[15]	3GPP TS 33.2:10: "Network Domain Security (NDS); IP network layer security".

[16]	IETF RFC 7515:"JSON Web Signature (JWS)".

[17]	IETF RFC 7519:"JSON Web Token (JWT)".

[18]	3GPP TS 23.501: "System Architecture for the 5G System".

[19]	3GPP TR 33.916: "Security Assurance Methodology (SECAM) for 3GPP network products".

*************** End of 3rd Change ****************

*************** Start of 4th Change ****************

### 4.4.1	Introduction

Basic Vulnerability Testing activities consist of requirements for running automated FOSS and COTS security testing tools against the external interfaces of a Network Product. These activities cover at least four aspects: Port Scanning, Vulnerability Scanner by the use of Vulnerability scanners and robustness/fuzz testing, and endpoint scanning. For each of these aspects, test requirements and test results are described in the present clause.

NOTE: 	The individual tools used for Basic Vulnerability Testing are selected by the evaluator. The SECAM accreditation body will ensure during accreditation of the evaluator's laboratory that the testers are able to utilize adequate tools.

*************** End of 4th Change ****************

*************** Start of 5th Change ****************

### 4.1.1	Pre-requisites for testing

The SCAS tests, as described in the present specification, are to be applied to a network product whose software and hardware has been brought into use so that the network product can provide the intended functionality, either in a real network environment or in a simulated environment. This implies that, before any testing is performed, the hardware and software has been installed correctly, the network product is powered on, and communication has been established over all standardized interfaces and O&M interfaces related with the network product's functionality, as described in the vendor's documentation.

##### 4.2.3.2.2	Protecting data and information – Confidential System Internal Data

Requirement Name: Unauthorized Viewing

Requirement Description: When the system is not under maintenance, there shall be no system function that reveals confidential system internal data in the clear to users and administrators. Such functions could be, for example, local or remote O&M CLI or GUI, logging messages, alarms, configuration file exports etc. Confidential system internal data contains authentication data (i.e. PINs, cryptographic keys, passwords, cookies) as well as system internal data that is not required for systems administration and could be of advantage to attackers (i.e. stack traces in error messages).

Security Objective references: tba.

Test case:

Test Name: TC_CONFIDENTIAL_SYSTEM_INTERNAL_DATA

Purpose:

Verify that no system function reveals sensitive data in the clear

Procedure and execution steps:

Pre-Condition:

The vendor shall provide documentation describing how confidential system internal information that could possibly be revealed in clear-text is handled by system functions.

A list of all system functions in the network product, information on how to enable and execute them should be provided as a part of the vendor's documentation. A system function is every function implemented in the network product needed by the services/functionalities provided by the network product itself.

Execution Steps

Execute the following steps:

1.	Review the documentation provided by the vendor describing how confidential system internal information is handled by system functions.

2.	The tester checks whether any system functions as described in the product documentation (e.g. local or remote O&M CLI or GUI, logging messages, alarms, error messages, configuration file exports, stack traces)  reveal any confidential system internal data in the clear (for example, passphrases).

Expected Results:

There should be no confidential system internal data revealed in the clear by any	 system function.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot containing the operational results.

##### 4.2.3.2.4	Protecting data and information in transfer

Requirement Name: Protecting data and information in transfer

Requirement Description:

-	Usage of cryptographically protected network protocols is required.

-	The transmission of data with a need of protection shall use industry standard network protocols with sufficient security measures and industry accepted algorithms. In particular, a protocol version without known vulnerabilities or a secure alternative shall be used.

Security Objective references: tba

Test case:

Test Name: TC_PROTECT_DATA_INFO_TRANSFER_1

Purpose:

Verify the mechanisms implemented to protect data and information in transfer to and from the Network Product's O&M interface.

NOTE: 	The test is limited to the O&M interface although the requirement does not have this limitation because the protection of standardised interfaces will be covered by regular interoperability testing and the proprietary use of HTTPS is covered in clause 4.2.5.1.

Procedure and execution steps:

Pre-Conditions:

Network product documentation containing information about supported O&M protocols is provided by the vendor,

A peer implementing the security protocol configured by the vendor (e.g SSH client supporting SSHv2 or HTTPS client) shall be available.

Network product documentation stating which security protocols for protection of data in transit are implemented and which profiles in TS 33.310 [9] and TS 33.210 [15] are applicable is provided by the vendor

For TLS/DTLS, the tester shall base the tests on the profile defined by 3GPP in TS 33.310 [9] and TS 33.210 [15]. For IKE and IPsec, the tester shall base the tests on the profile defined by 3GPP in TS 33.210 [15]. For protocols, for which 3GPP did not define a security profile, e.g. SSH, the tester shall base the tests on a widely recognised and publicly available security profile.

4.2.3.4.1.2	Accounts shall allow unambiguous identification of the user.

Requirement name: The network product shall use accounts that allow unambiguous identification of the user.

Requirement Description: Users shall be identified unambiguously by the network product. The network product shall support assignment of individual accounts per user, where a user could be a person, or, for Machine Accounts, an application, or a system. The network product shall not enable the use of group accounts or group credentials, or sharing of the same account between several users, by default. The network product shall support a minimum number of 50 individual accounts per user data base if not explicitly specified in a SCAS of a particular network product, so that accountability for each user is ensured even in large operator networks. The network product shall not support user access credentials unrelated to an account.

NOTE 1:	The network product may support independent user data bases for different access methods, e.g. one data base for command shell access on OS level and another data base for GUI access. User data bases may be stored locally on the network product or on a central AAA system that the network product accesses for user authentication.

NOTE 2:	This requirement does not preclude user group concepts for access control.

Security Objective references: tba.

Test case:

NOTE 3:	Some typical default accounts suggest that they are shared amongst several persons (e.g. vendor_xy, support), or do not allow identification of individual users (e.g. guest, ftp, anonymous). In order to avoid overlap of this test case with clause 4.2.3.4.2.2, it is assumed for this test case that such accounts have been deleted or disabled in line with clause 4.2.3.4.2.2.

Test Name: TC_ACCOUNT_DOCUMENTATION

Purpose:

To ensure that documentation of the network product does not encourage or require the use of group accounts, group credentials, or sharing of the same account between several users. To ensure that the network product does not support credentials unrelated to an account.

Procedure and execution steps:

Pre-Conditions:

1)	All user and group data bases for names and credentials supported by the network product are identified in the documentation accompanying the network product.

2)	All predefined accounts and groups are identified in the documentation accompanying the Network Product.

3)	Instructions of how administrator users can add accounts, groups, and credentials to the database(s) are provided in the documentation accompanying the Network Product.

4) 	The operations manual describes O&M user and group concepts supported by the network product.

Execution Steps:

The accredited evaluator's test lab is required to execute the following steps:

1)	Review the system documentation (in particular operations manual) whether it encourages or requires the use of group accounts, group credentials, or sharing of the same account between several users.

2)	Review the system documentation whether the network product requires or supports entering credentials unrelated to an account, in order to perform specific actions, e.g. to enter a "master password" for access to privileged functions.

4.2.3.4.6.2	Role-based access control

Requirement Name: tba

Requirement Description:

The network product shall support Role Based Access Control (RBAC). A role-based access control system uses a set of controls which determines how users interact with domains and resources. The domains could be Fault Management (FM), Performance Management (PM), System Admin, etc. The RBAC system controls how users or groups of users are allowed access to the various domains and what type of operation they can perform, i.e. the specific operation command or command group (e.g. View, Modify, Execute).

The network product supports RBAC, in particular, for O&M privilege management for network product Management and Maintenance, including authorization of the operation for configuration data and software via the network product console interface.

##### 4.2.3.5.2	Protecting sessions – Inactivity timeout

Requirement Name: Protecting sessions – inactivity timeout

Requirement Description: An O&M user interactive session shall be terminated automatically after a specified period of inactivity. It shall be possible to configure an inactivity time-out period.

Note: 	The kind of activity required to reset the timeout timer depends on the type of user session.

Test Name: TC_PROTECTING_SESSION_ INAC TIMEOUT

Purpose:

To ensure an O&M user interactive session shall be terminated at inactivity timeout.

Procedure and execution steps:

Pre-Conditions:

-	The tester has privileges to create an O&M user interactive session.

-	The tester has privileges to configure the inactivity time-out period for user interactive session.

-	Session log should be enabled.

Execution Steps

1.	The tester creates O&M user A interaction session.

2.	The tester configures the inactivity time-out period for user A to x minute, for example 1 minute.

3.	The tester does not make any actions on the network production in x minutes. After that, the tester checks whether O&M user A interaction session has been terminated automatically.

Expected Results:

-	In step 3, O&M user A interaction session has been terminated automatically after x minute.

*************** End of 5th Change ****************