3GPP TSG-SA3 Meeting #113 	S3-234422

# Chicago, US, 6 - 10 November 2023											            revision of S3-23yyyy

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0130 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Evidence correction for 33.117 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Deutsche Telecom, T-mobile, ZTE, Nokia, Ericsson, China Mobile, Federal Office for Information Security (BSI), TELUS, MITRE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 4.2.3.4.2.1 and 4.2.3.4.5, the expected format of evidence is “TBA”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Supplement for expected format of evidence in clause 4.2.3.4.2.1 and 4.2.3.4.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3.4.2.1, 4.2.3.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of 1st Change ****************

4.2.3.4.2.1	Account protection by at least one authentication attribute.

Requirement Name: Account protection by at least one authentication attribute.

Requirement Description: The various user and machine accounts on a system shall be protected from misuse. To this end, an authentication attribute is typically used, which, when combined with the user name, enables unambiguous authentication and identification of the authorized user.

Authentication attributes include:

-	Cryptographic keys

-	Token

-	Passwords

This means that authentication based on a parameter that can be spoofed (e.g. phone numbers, public IP addresses or VPN membership) is not permitted. Exceptions are attributes that cannot be faked or spoofed by an attacker.

NOTE: 	Several of the above options can be combined (dual-factor authentication) to achieve a higher level of security. Whether or not this is suitable and necessary depends on the protection needs of the individual system and its data and is evaluated for individual cases.

Security Objective references: tba.

TEST CASE:

Test Name: TC_ACCOUNT_PROTECTION

Purpose:

To ensure that all accounts are protected by at least one authentication attribute.

Procedure and execution steps:

Pre-Conditions:

1)	All predefined accounts are identified in the documentation accompanying the Network Product.

2)	Instructions of how to create new accounts are provided in the documentation accompanying the Network Product.

3)	Instructions of how administrator user can view all existing accounts in the database are provided in the documentation accompanying the Network Product.

NOTE: 	No test is provided here for finding undocumented hard coded accounts as such tests may be impossible to define in a general way.

Execution Steps:

The accredited evaluator's test lab is required to execute the following steps:

1)	After login via account with necessary access rights (e.g. Admin) search in the database for any undocumented account.

2)	Attempt login to all predefined accounts identified (either documented or not) with and without using the respective authentication attribute.

3)	Create a new account by following instructions in documentation.

4)	Attempt login to the newly created account.

Expected Results:

1)	It is not possible to login to any predefined account without using at least one authentication attribute that satisfies the conditions in the requirement.

2)	It is not possible to login to any newly created account without usage of at least one authentication attribute that satisfies the conditions in the requirement.

Expected format of evidence:

Evidence containing the results of all login attempts, e.g. screenshots, log files, error messages.

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

4.2.3.4.5	Policy regarding consecutive failed login attempts

Requirement Name: tba

Requirement Description:

a) 	The maximum permissible number of consecutive failed user account login attempts should be configurable by the operator. The definition of the default value set at manufacturing time for maximum number of failed user account login attempts shall be less than or equal to 8, typically 5. After the maximum permissible number of consecutive failed user account login attempts is exceeded by a user there shall be a block delay in allowing the user to attempt login again. This block delay and also the capability to set period of the block delay, e.g. double the delay, or 5 minutes delay, or 10 minutes delay, after each login failure should be configurable by the operator. The default value set at manufacturing time for this delay shall be greater than or equal to 5 sec.

b) 	If supported, infinite (permanent) locking of an account that has exceeded maximum permissible number of consecutive failed user account login attempts should also be possible via configuration, with the exception of administrative accounts which shall get only temporarily locked.

Security Objective references: tba.

TEST CASE:

Test Name: TC_FAILED_LOGIN_ATTEMPTS

Purpose:

To ensure that the policy regarding failed login attempts is adhered to.

Case 1: Testing for requirement 4.2.3.4.5 a)

Procedure and execution steps:

Pre-Conditions:

1)	At least one user account has been created as per manufacturer's instructions.

2)	Directions of how to configure the maximum permissible number of consecutive failed user account login attempts and the default value of this number are identified in the documentation accompanying the Network Product. Default value shall be stated as well.

3)	Directions of how to configure the block delay in allowing a user attempt to login again when the number of failed login attempts has exceeded the maximum number are identified in the documentation accompanying the Network Product. Default value of the delay shall be stated as well.

Execution Steps:

The accredited evaluator's test lab is required to execute the following steps:

1)	Check default values from precondition 2 and 3.

2)	Perform consecutive failed login attempts for the user account until the default maximum number of precondition 2 is reached.

3)	Attempt again one extra login, which fails again.

4)	Attempt one extra login in less time than the default for the delay of precondition 3, using the correct credentials.

5)	Attempt one extra login in more time than the default for the delay of precondition 3, using the correct credentials.

Expected Results:

1)	Default values from precondition 2 and 3 are in accordance with the requirement.

2)	In execution step 4, the login attempt shall be rejected in all cases.

3)	In execution step 5, the login attempt shall be accepted.

4)	In execution step 6, it is verified that the user can login only at the last login attempt.

Expected format of evidence:

Evidence containing the results of all login attempts, e.g. screenshots, log files, configurations, error messages.

Case 2: Testing for requirement 4.2.3.4.5 b)

Procedure and execution steps:

Pre-Conditions:

1. 	At least one user account has been created as per manufacturer's instructions.

2. 	Directions of how to configure the maximum permissible number of consecutive failed user account login attempts and the default value of this number are identified in the documentation accompanying the Network Product. Default value shall be stated as well.

3. 	Directions of how to optionally configure permanent locking for non-administrative accounts shall be stated as well.

Execution Steps:

The accredited evaluator's test lab is required to execute the following steps:

1. 	Check default values from precondition 2.

2. 	Perform consecutive failed login attempts for the user account until the default maximum number of precondition 2 is reached.

3. 	Attempt again one extra login, which fails again.

4. 	Attempt one extra login in more time than the default for the delay of precondition 3, using the correct credentials.

5a. 	If supported enable permanent locking of accounts exceeding the maximum permissible number of consecutive failed user account login attempts and repeat steps 1-4 for a normal user.

5b. If supported enable permanent locking of accounts exceeding the maximum permissible number of consecutive failed user account login attempts and repeat steps 1-4 for a user with administrative access rights.

Expected Results:

In execution step 5a it is verified that the user cannot login at any execution step.

In execution step 5b it is verified that an administrator user can successfully login only at execution step 5b.

Expected format of evidence:

Evidence containing the results of all login attempts, e.g. screenshots, log files, configurations, error messages.

*************** End of 2nd Change ****************