3GPP TSG-SA3 Meeting #113 	S3-234449

# Chicago, US, 6 - 10 november 2023

# Source:	3GPP CT1

# Title:	LS on procedures for UE discovery for Ranging_SL

# Document for:	Information, Discussion

Agenda Item:  3

3GPP TSG-CT WG1 Meeting #141e	C1-236527

# Online 17– 21 April 2023

# Title:	LS on procedures for UE discovery for Ranging_SL

# Response to:

# Release:	Release 18

# Work Item:	5G_ProSe

Source:	CT1

To:	SA2

Cc:	SA3

Contact Person:

Name:	Tingfang Tang

Tel. Number:

E-mail Address:	tangtingfang(at)xiaomi(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT1 has found the following understanding on UE discovery for Ranging/Sidelink Positioning UE discovery with 5G ProSe capable UE:

- The procedures for Ranging/Sidelink Positioning UE discovery with 5G ProSe capable UE uses both Model A discovery and Model B discovery are defined in clause 6.3.2 of TS 23.304;

- The DDNMF is not included in the architecture, then the ProSe Application Code or ProSe Restricted Code for Model A, and ProSe Query Code/ProSe Response Code for Model B retrived from DDNMF are not used in the procedrues of Ranging/SL Positioning UE discovery with 5G ProSe capable UE as defined in clasue 6.4.2 of TS 23.586;

- the “discovery type” is introduced to indicate "Ranging/Sidelink Positioning", but there’s no description on how this information is used in the UE discovery procedrue;

- In clause 11.2.1 of 3GPP TS 24.554 under CT1 control and in 3GPP TS 23.303 under SA2 control, the “Discovery type” within ProSe direct discovery PC5 message type was already there indicating one of the following for the discovery message:

- Open discovery;

- Restricted discovery.

It can be observed that

- the exising “Discovery type” descibes the open or restricted discovery type, its meaning is different from the “discovery type” introduced for ranging_SL in SA2 to indicate "Ranging/Sidelink Positioning". It is not suitable to extend the existing“discovery type” to support Ranging/Sidelink Positioning UE discovery procedure.

Considering the above, clarification on the UE discovery for Ranging/Sidelink Positioning UE discovery with 5G ProSe capable UE is needed, CT1 would like to ask SA2:

- Whether the “discovery type” indicating "Ranging/Sidelink Positioning" described in SA2 is a different information from the  the existing “discovery type” indicating the open or restricted discovery type or the same one?

- If the answer is no, could SA2 consider to define a new one to differentiate it from the existing “discovery type” indicating the open or restricted discovery type?

2. Actions:

To SA WG2 group.

ACTION: 	CT WG1 kindly asks SA WG2 group to provide CT WG1 feedback for the above questions.

3. Date of Next CT1 Meetings:

CT1#144	9th   – 13th October 2023 		China (TBC)

CT1#145	13th   – 17th November 2023 		Chicago

CT1#146	22nd  – 26th January 2024 		TBD