3GPP TSG-SA3 Meeting #113 	S3-234484

# Chicago, US, 6 - 10 November 2023

# Source:	3GPP SA2

# Title:	LS on the progress of 5WWC_Ph2 normative work

# Document for:	Information, Discussion

Agenda Item:  3

SA WG2 Meeting #159	S2-2311801

# Xiamen, PRC, October 9-13th, 2023        	  		         (revision of S2-2311494)

Title:	LS on the progress of 5WWC_Ph2 normative work

Response to:	BBF (LIAISE-562-05 / S2-2306266) and CableLabs LS Response on 3GPP Rel-18 5WWC_Ph2 (S2-2306267)

Release:	Rel-18

Work Item:	5WWC_Ph2

Source:	SA2

To:	BBF, CableLabs

Cc:	CT1, CT3, CT4, SA3

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2311802 (CR 2115 to 23.316)

1. Overall Description:

SA2 thanks BBF and CableLabs for their LS on solutions for 5WWC_Ph2 Key Issue 1 which it has used to reach its conclusions.

SA2 would like to inform BBF, CableLabs that it has completed its normative work on 5WWC_Ph2 and that no further functional addition can be made in 3GPP release 18. Normative specifications related with 5WWC_Ph2 Key Issue 1 are defined in TS 23.316 v18.3.0 (and more recent versions) mostly in clauses 4.10, 4.10b, 4.10c, 4.10d but also in clauses e.g. 4.5.3, 7.2.8, 7.3.8, 9.8, 4.7.11, 9.5.2.1 (this is not an exhaustive clause list); The main functionalities are:

- Support 5G non-seamless Wireless offload NSWO for 3GPP UE accessing via 5G-RG

- the support for device that does not support NAS signaling, which is connected to 5GC via a RG and can be authenticated by 5GC over the RG, called AUN3 device.

- the support for  device that does not support NAS signalling which is connected to 5GC via a RG and for which authentication with 5GC is not supported, called NAUN3 device

In order to complete the specification of these features, BBF and CableLabs should, if possible, define aspects under their responsibility, including:

- In order to support 5G NSWO the Wireline access shall support the SWa' interface (toward the NSWOF). This specification including how the offloaded traffic is carried in W-5GAN and bypass the 5GC of the UE

- The support of AUN3 device requires that the 5G-RG acts on behalf of the AUN3 device supporting UE functionalities, for example the NAS termination on behalf of the AUN3 , the PDU session establishment, the URSP policy, etc. 
How the 5G-RG is triggered to apply procedures for AUN3 devices is assumed to be defined by BBF and/or CableLabs.
The W-AGF control on the maximum bit rate for the aggregated traffic of the 5G-RG and of the AUN3 devices served by this 5G-RG based on the RG Level Wireline Access Characteristics received from 5GC over N2, which is extended (see attached CR)

2. Actions:

To BBF, CableLabs:

ACTION: 	SA2 kindly asks BBF, CableLabs to provide any questions, feedback you have on Normative specifications related with R18 5WWC_Ph2.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | country |
| --- | --- | --- | --- |
| SA2#160 | November 13th – 17th, 2023 | Chicago | USA |
| SA2#160 AH | January 22-26th, 2024 | TBD | TBD |
| SA2#161 | February 26th – March 1st, 2024 | EU | EU |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/