3GPP TSG-SA3 Meeting #113 	S3-234520

# Chicago, US, 6 - 10 November 2023

Title:	[Draft] Reply LS on Retrieving keys for decryption of protected IEs for U2N relay

Response to:	LS S3-234453/C1-234362 on LS on Retrieving keys for decryption of protected IEs for U2N relay from CT1

Release:	Rel-17

Work Item:	5G_ProSe

Source:	Interdigital (to be SA3)

To:	CT1

Cc:

Contact person:	Samir Ferdi

Samir dot Ferdi at interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	<Attachment in S3-234521>

# 1	Overall description

SA3 would like to thank CT1 for their LS on Retrieving keys for decryption of protected IEs for U2N relay.

Question from CT1 in C1-234362:

Question:	How does the 5G ProSe UE-to-network relay UE retrieve the corresponding discovery security parameters for decryption of the relay service code and the UP-PRUK ID/CP-PRUK ID in the PROSE DIRECT LINK ESTABLISHMENT REQUEST message? Is there any additional security mechanism needed to be defined by SA3?

Answer from SA3:

SA3 has acknowledged the issue from CT1 and agreed to the following solution (captured in the attached CR):

- During the U2N Relay discovery procedure, the U2N Relay provides the authorized Remote UE with a protected DCR Security ID (DSID) associated with the RSC.

- The U2N Relay uses the DSID included in the DCR to locate the discovery security keys to decrypt the RSC, UP-PRUK ID/CP-PRUK ID in the DCR.

# 2	Actions

To CT1

ACTION: 	SA3 kindly asks CT1 to take the above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#115	February 26th – March 1st 2024	Athens, Greece

SA3#116	May 20th – 25th 2024		Korea, KR