3GPP TSG-SA3 Meeting #113 	S3-234524

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0181 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction in UDM and GPSI related requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | GPSI was introduced in the last meeting, however, GPSI related aspect were not fully aligned in the specs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AAnF and UDM interface will be confidentiality, integrity and replay protected. AAnF needs to retrieve the GPSI from UDM that service is defined in 23.502 (33501 does not define the Nudm_SDM_Get service) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.1, 4.3.0, 4.4.0 , 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES***********

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[5]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[6]	IETF RFC 7542: "The Network Access Identifier".

[7]	3GPP TS 33.222: " Generic Authentication Architecture (GAA); Access to network application functions using HypertextTransfer Protocol over Transport Layer Security (HTTPS)".

[8]	Void

[9]	3GPP TS 23.003: "Numbering, addressing and identification".

[10]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[11]	3GPP TS 29.503: "5G System; Unified Data Management Services ".

[12]	IETF RFC 9146: "The Datagram Transport Layer Security (DTLS) Protocol Version 1.3"

[13]	3GPP TS 33.210: "3G Security; Network Domain Security; IP network layer security".

[14]	IETF RFC 8613: "Object Security for Constrained RESTful Environments (OSCORE)".

[15]	IETF RFC 8949: "Concise Binary Object Representation (CBOR)".

[16]	IETF RFC 5869: "HMAC-based Extract-and-Expand Key Derivation Function (HKDF)".

[x]	3GPP TS 23.502: "Procedures for the 5G System".

************ NEXT CHANGES***********

### 4.2.1	AAnF

The AAnF is the anchor function in the HPLMN. The AAnF stores the AKMA Anchor Key (KAKMA) and SUPI/GPSI for AKMA service, which is received from the AUSF/UDM after the UE completes a successful 5G primary authentication. The AAnF also generates the key material to be used between the UE and the Application Function (AF) and maintains UE AKMA contexts. The AAnF sends SUPI/GPSI of the UE to AF located inside the operator's network according to the AF request or sends SUPI to NEF. If GPSI is required, the AAnF may retrieve the GPSI from UDM based on available SUPI. The AAnF has the capability to trigger a primary authentication for KAKMA refreshing purpose.

************ NEXT CHANGES***********

### 4.3.0	General

The following interfaces are involved in AKMA network architecture:

-	Nnef: Service-based interface exhibited by NEF.

-	Nudm: Service-based interface exhibited by UDM.

NOTE 1:	UDM services related to AKMA service are defined in TS 33.501 [2] clause 14.2.2 ,14.2.6 and in TS 23.502[x] clause 5.2.3.3.2 .

-	Naanf: Service-based interface exhibited by AAnF.

The AAnF interacts with the AUSF and the AF using Service-based Interfaces. When the AF is located in the operator's network, the AAnF shall use Service-Based Interface to communicate with the AF directly. When the AF is located outside the operator's network, the NEF shall be used to exchange the messages between the AF and the AAnF.

************ NEXT CHANGES***********

## 4.4.0	General

The following security requirements are applicable to AKMA:

-	AKMA shall reuse the same UE subscription and the same credentials used for 5G access.

-	AKMA shall reuse the 5G primary authentication procedure and methods specified in TS 33.501 [2] for the sake of implicit authentication for AKMA services.

-	The SBA interface between the AAnF and the AUSF shall be confidentiality, integrity and replay protected.

-	The SBA interface between AAnF and AF/NEF shall be confidentiality, integrity and replay protected.

-	The SBA interface between AAnF and UDM shall be confidentiality, integrity and replay protected.

-	The AKMA Application Key (KAF) shall be provided with a maximum lifetime based on the operator’s local authentication policy.

NOTE:	Void

## 7.4	Services provided by UDM

UDM services related to AKMA service are defined in TS 33.501 [2] clause 14.2.2,14.2.6 and in TS 23.502[x] clause 5.2.3.3.2.

************ END OF CHANGES**********