3GPP TSG-SA3 Meeting #113 	S3-234530

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0186 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AKMA service restriction in VPLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Please refer to the discussion paper S3-234529 As per the AKMA TS 33535 Rel 18 4.6.1 AKMA roaming requirements - The roaming subscriber shall be able to utilize the AKMA feature provided by the home network. - The home network shall be able to control whether its subscriber is authorized to use the service in the visited network. Definitely, UE can access AKMA/AF over non-3GPP access, as asked in the LS. TS 33.535 does not contain any restrictions. As CT3 indicated in their LS, when UE is in a visiting country and tries to connect to AF (AKMA), then: - When the UE establish PDU session via 3GPP access, VPLMN AMF/SMF is selected, and PDU session is treated as roaming. - When the UE establish PDU session via Non-3GPP access, HPLMN AMF/SMF is selected, and PDU session is treated as non-roaming. i.e. UE is in a visiting country but connected via a non-3GPP access network where selected N3IWF and AMF are in HPLMN, then it is considered a non roaming scenario. In this case, all non-roaming policies and configurations will be applicable to UE, and accordingly, non-roaming charging/billing will be applicable to UE. In the case of AKMA too (as mentioned in the LS), if UE is connected via this non-3GPP access (HPLMN AMF), then AKMA service should be allowed because it is considered an HPLMN scenario. The only thing is that 5GC needs to determine if UE is connected to AF via non-3GP access or non-3GPP access. The SA2/CT group already defines a procedure to determine the access type of the UE associated with the UE IP address. So we can leverage these aspects to determine the access typ, check the serving PLMN ID, and allow or restrict the UE accessing the AKMA., |  |  |  |  |  |  |  |  |
| Summary of change: |  | AF provides the UE IP address associated with the AF. AANF uses the UE IP address to determine the UE access type via existing means. E.g.AAnF uses accessType to determine the associated PLMN ID via the existing UDM API and determines the roaming or non-roaming status of UE in accordance with the UE connection. Apply operator policy to allow or restrict UE access to AKMA service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not apply operator policy deterministically to restrict or allow the UE AKMA service in roaming. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1, 6.3, 7.1.3, 7.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES*****

### 6.2.1	AAnF response with UE Identity

Figure 6.2-1 shows the procedure used by the AF to request application function specific AKMA keys from the AAnF, when the AF is located inside the operator's network.

Figure 6.2-1: KAF generation from KAKMA

Before communication between the UE and the AKMA AF can start, the UE and the AKMA AF need to know whether to use AKMA. This knowledge is implicit to the specific application on the UE and the AKMA AF or indicated by the AKMA AF to the UE (see clause 6.5).

1.	The UE shall generate the AKMA Anchor Key (KAKMA) and the A-KID from the KAUSF before initiating communication with an AKMA Application Function. When the UE initiates communication with the AKMA AF, it shall include the derived A-KID (see clause 6.1) in the Application Session Establishment Request message. The UE may derive KAF before sending the message or afterwards.

2.	If the AF does not have an active context associated with the A-KID, then the AF selects the AAnF as defined in clause 6.7, and sends a Naanf_AKMA_ApplicationKey_Get request to AAnF with the A-KID to request the KAF for the UE. The AF also includes its identity (AF_ID) in the request.

AF_ID consists of the FQDN of the AF and the Ua* security protocol identifier (see Annex A.4). The latter parameter identifies the security protocol that the AF will use with the UE.

The AAnF shall check whether the AAnF can provide the service to the AF based on the configured local policy or based on the authorization information available in the signalling (i.e., Oauth2.0 token). If it succeeds, the following procedures are executed. Otherwise, the AAnF shall reject the procedure.

The AAnF shall check if UE IP address is associated with 3GPP access or non-3GPP access via the existing procedure available at BSF and SMF/PCF.

NOTE: AAnF utilizes BSF and SMF/PCF existing capability to determine the access type associated with the UE IP address. i.e. AAnF retrieves the service PCF/SMF details from the BSF based on UE IP address. If NATing is used, AAnF determines the public/private IP address mapping via procedure defined in clause 4.15.10 of TS 23.502. Then AAnF retrieves the access type associated with UE IP address from PCF/SMF

The AAnF shall verify whether the subscriber is authorized to use AKMA based on the presence of the UE specific KAKMA key identified by the A-KID.

If KAKMA is present in AAnF, the AAnF shall continue with step 3.

If KAKMA is not present in the AAnF, the AAnF shall continue with step 4 with an error response.

3.	Once receiving the request from the AF, the AAnF retrieves the 3gpp AMF registration data from UDM via Nudm_UECM_Get if the access type associated with UE is 3GPP access. Otherwise, AAnF retrieves the non-3gpp AMF registration data. If the AAnF determines this specific AF needs GPSI, according to its local policy, the AAnF also sends a Nudm_SDM_Get Request to the UDM to fetch the GPSI of the UE.

4.	The UDM responds with the AMF registration data that included registered PLMN and associated access type and optionally GPSI of the UE if requseted. The AAnF shall store the received GPSI as part of UE’s AKMA context.

AAnF determines that UE is connected to AF via VPLMN or HPLMN based on the PLMN ID available in the amf registration data. Based on the operator policy on whether AKMA service is allowed in VPLMN or not, AAnF may allow or reject the AF request.

5.	The AAnF derives the AKMA Application Key (KAF) from KAKMA if it does not already have KAF.

The key derivation of KAF shall be performed as specified in Annex A.4.

6.	The AAnF sends Naanf_AKMA_ApplicationKey_Get response to the AF with SUPI, KAF and the KAF expiration time. Whether to send SUPI or GPSI is determined by AAnF based on the local policy.

7.	The AF sends the Application Session Establishment Response to the UE. If the information in step 4 indicates failure of AKMA key request, the AF shall reject the Application Session Establishment by including a failure cause. Afterwards, UE may trigger a new Application Session Establishment request with the latest A-KID to the AKMA AF.

************ NEXT CHANGES**********

## 6.3	AKMA Application Key request via NEF

Figure 6.3-1 shows the procedure used by the AF to request KAF from the AAnF via NEF, when the AF is located outside the operator's network.

Figure 6.3-1: AKMA Application Key request via NEF

1.	When the AF is about to request AKMA Application Key for the UE from the AAnF, e.g. when UE initiates application session establishment request as in clause 6.2.1, the AF discovers the HPLMN of the UE based on the A-KID and sends the request towards the AAnF via NEF service API. The request shall include the A-KID, UE IP address, and the AF_ID and optionally UE Id not needed indication.

NOTE:	In the case of architecture without CAPIF support, the AF is locally configured with the API termination points for the service. In the case of architecture with CAPIF support, the AF obtains the service API information from the CAPIF core function via the Availability of service APIs event notification or Service Discover Response as specified in TS 23.222 [5].

2.	If the AF is authorized by the NEF to request KAF, including the authorization after verification of the AF_ID in step 1, the NEF discovers and selects an AAnF as defined in clause 6.7.

3.	The NEF sends a Naanf_AKMA_ApplicationKey_Get request to the selected AAnF with the A-KID to request the KAF for the UE.

The AAnF shall process the request in the same way as specified in clause 6.2.1 with following changes:

If KAKMA is present in AAnF, the AAnF shall continue with step 4 in this clause.

If KAKMA is not present in the AAnF, the AAnF shall continue with step 5 in this clause with an error response.

4.	The AAnF generates the KAF as specified in clause 6.2.1 and sends the response to the NEF with the KAF, the KAF expiration time (KAF exptime) and SUPI.

5.	The NEF forwards the response to the AF with the KAF, the KAF expiration time (KAF exptime) and optionally GPSI (external ID). Based on local policy, the NEF uses the Nudm_SubscriberDataManagement service which is specified in TS 29.503[11] to translate SUPI to GPSI (external ID) and optionally include GPSI (external ID) in the response. If UE Id not needed indication is received in the incoming request, the NEF shall not provide the GPSI (external ID) to AF. The NEF shall not send the SUPI to the AF.

************ NEXT CHANGES**********

### 7.1.3	Naanf_AKMA_ApplicationKey_Get service operation

Service operation name: Naanf_AKMA_ApplicationKey_Get.

Description: The NF consumer requests AKMA Application Key and UE ID from the AAnF.

Input, Required: A-KID, AF_ID , UE IP address.

Input, Optional: None.

Output, Required: KAF, KAF expiration time and SUPI or GPSI.

Output, Optional: None.

************ NEXT CHANGES**********

### 7.1.5	Naanf_AKMA_ApplicationKey_ AnonUser_Getservice operation

Service operation name: Naanf_AKMA_ApplicationKey_AnonUser_Get.

Description: The NF consumer requests only the AKMA Application Key from the AAnF. This service is for allowing anonymous user access to the AF based on A-KID (i.e., UE identification is not required at the AF). The A-KID functions as a temporary user identifier.

Input, Required: A-KID, AF_ID, UE IP address.

Input, Optional: None.

Output, Required: KAF, KAF expiration time.

Output, Optional: None.

### 7.3.2	Nnef_AKMA_ApplicationKey_Get service operation

Service operation name: Nnef_AKMA_ApplicationKey_Get.

Description: The NF consumer requests the NEF to provide AF related key material.

Input, Required: A-KID, AF_ID, UE IP address.

Input, Optional: UEID not needed indication.

Output, Required: KAF, KAF expiration time.

Output, Optional: GPSI (external ID).

************ END OF CHANGES**********