3GPP TSG-SA3 Meeting #113 	S3-234544

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1801 | rev | - | Current version: | 18.03.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Framework for NF accessing the external AF data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Please refer to the discussion paper S3-234542 There are requirements defined in SA2 (TS 23.288/NWDAF specs) and SA3 where if NWDAF wants to collect data from external AF, the NWDAF needs to request NEF and NEF collects data from AF and provides the data back to NF. However, there is no clarification regarding the security aspect of how NF should or can request data from external AF via NEF in chapters 12 and 13. Therefore, clarification should be added on how NF should collect the data from external AF and how authentication and authorization will work in this case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification is added regarding the authorization of NF requesting data from AF via NEF. Typo corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. It is not clear how NF should be allowed to access data from external AF and how the authorization aspect will work in this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.2.3, 12.1, 12.4, 12.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st CHANGE ***

#### 5.9.2.3	NEF security requirements

The Network Exposure Function (NEF) supports external exposure of capabilities of Network Functions to Application Functions, which interact with the relevant Network Functions via the NEF. NEF is also used when Network Functions wants to consume the services offered by external Application Functions.

The interface between the NEF and the Application Function shall fulfil the following requirements:

-	Integrity protection, replay protection and confidentiality protection for communication between the NEF and Application Function shall be supported.

-	Mutual authentication between the NEF and Application Function shall be supported.

-	Internal 5G Core information such as DNN, S-NSSAI etc., shall not be sent outside the 3GPP operator domain.

-	SUPI shall not be sent outside the 3GPP operator domain by NEF.

The NEF shall be able to determine whether the Application Function is authorized to interact with the relevant Network Functions. The NEF shall be able to determine whether the Network Function is authorized to interact with the external Application Function associated with the NEF.

*** NEXT CHANGE ***

## 12.1	General

In the 5G system, the Network Functions securely expose capabilities and events to 3rd party Application Functions (AF) via NEF. Additionally, NEF is used when Network Functions wants to consume the services offered by 3rd party Application Functions. The NEF also enable secure provision of information in the 3GPP network by authenticated and authorized AFs.

NOTE:	If a token is generated for AF authorization, such a token can include specific information depending on the procedure, e.g. clause 16.6.3.

Requirements on security aspects of NEF are captured in clause 5.9.2.3.

*** NEXT CHANGE ***

## 12.4	Authorization of Application Function’s requests

After the authentication, NEF determines whether the AF is authorized to send requests for the 3GPP Network Entity. The NEF shall authorize the requests from AF using OAuth-based authorization mechanism, the specific authorization mechanisms shall follow the provisions given in RFC 6749 [43].

*** NEXT CHANGE ***

## 12.x	Authorization of Network Function’s requests for accessing the AF

After the authentication, the NEF determines whether the NF is authorised to send requests for external AF. The NEF shall authorise the requests from NF using an OAuth-based authorization mechanism or authorization policy as defined in clause13.3.

*** END OF CHANGE ***