3GPP TSG-SA3 Meeting #113 	S3-234551

# Chicago, US, 6 - 10 November 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Discussion paper on adding X.509 trust anchor by CA

# Document for:	Discussion

Agenda Item:	4.1.15

# 1	Decision/action requested

It is requested to endorse the proposal of this discussion paper.

# 2	References

[1]	3GPP TS 33.310 “Network Domain Security (NDS); Authentication Framework (AF)”

[2]	3GPP TR 33.876 “Study on automated certificate management in Service-Based Architecture (SBA)”

[3]	IETF RFC 4210 “Internet X.509 Public Key Infrastructure Certificate Management Protocol (CMP)”

# 3	Rationale

In different scenarios in RAN it is required the deployment of additional trust anchors/Root CA certificates to base stations, i.e., it may be required the configuration of multiple Root CAs in their ‘trust’ pool. Some of those scenarios are among others:

- RAN sharing: The establishment of IPsec connections to RAN sharing partners with their own PKI, i.e., a BTS may belong to a RAN sharing network where the security gateways of each partner operator belong to different Root CA than the shared BTS.

- Multiple CA hierarchies: Operator PKI may need to be segregated into multiple Root CA hierarchies based on the different network domains, type of network element/function, vendor, geographical areas, etc.

  - Example: BTS may have their own Root CA, Security Gateways may have their own Root CA, and OAM system their own Root CA. As a consequence, the BTS needs to be manually preconfigured with all those Root CA certificates in case of a new site or after a hardware swap.

Observation 1: Current 3GPP SA3 specification TS 33.310 [1] in clauses 9.5.2 and 9.5.4 requires the support of extraCerts field in the CMPv2 profiling for base stations:

“The extraCerts field of the PKIMessage carrying the initialization response shall be mandatory and shall contain the operator root certificate and the RA/CA certificate (or certificates if separate private keys are used for signing of certificates and CMP messages). If the RA/CA certificate(s) are not signed by the operator root CA, also the intermediate certificates for the chain(s) up to the operator root certificate shall be included in the extraCerts field.”

Observation 2: extraCerts field is not under CMP message integrity protection, whereas caPubs field (under CMP message integrity protection as described in RFC4210 [3]) has not been specified in clause 9 of TS 33.310 (Certificate enrolment for base stations) for transporting root CA certificate.

Observation 3: in contrast, CMPv2 profiling for 5G SBA Network Functions has specified the optional use of caPubs field in clause 10.3.1.4.4 of TS 33.310:

The extraCerts field of the PKIMessage carrying the initialization response shall be mandatory and shall contain the operator root certificate (or ‘full chain’ if NF contacted to SubCA using CMPv2) and the RA/CA certificate (or certificates if separate private keys are used for signing of certificates and CMP messages). If the RA/CA certificate(s) are not signed by the operator root CA, also the intermediate certificates for the chain(s) up to the operator root certificate shall be included in the extraCerts field. If additional (self-signed) Root CA certificates are required, they shall be carried in the extraCerts field or caPubs field of the PKIMessage. Since extraCerts field is not under CMP message integrity protection, CMP over TLS should be used as a security transport mechanism. Since CMP already supports integrity protection for caPubs field, the use of security transport mechanisms is optional.

Problem statement 1: There is not a standardized mechanism to download automatically via enrolment protocol (in-band) other trust anchor root CA certificate than the BTS/Network Function own trust chain in extraCerts field. I.e., additional trust anchors/Root CA certificates are to be manually preconfigured.

Problem statement 2: Since extraCerts field is not under CMP protection, adding in this field self-signed root CA certificates can bring the risk in weak implementations of taking them as trusted certificates. I.e, it can be manipulated in-line by RA or proxies, so a potential attacker (man-in-the-middle) can relatively easily add fake Root CA certificates, which would be a severe security breach.

# 4	Detailed proposal

Two alternative solutions based on CMPv2 are proposed to solve the referred problems of lack of automation in the deployment of additional required Root CA certificates (trust anchors), and the potential security issues in the use of extraCerts field for transporting Root CA certificates. Both solutions would modify the existing CMPv2 profile for BTS, and corresponding CR (S3-23xxxx) has been proposed for that.

The following diagram represents graphically the scenario for both solutions:

Figure 1: Adding root CA certificates via CMP

Proposal 1: Extended usage of ‘extraCerts’ field

The CA which issues 3GPP operator base station certificates via CMPv2 can send additional Root CA certificates within the ‘ip’ (initialization response) or ‘kur’ (key update response) message as information element within the field ‘extraCerts’ to the corresponding BTS. The BTS simply needs to copy those additional Root CA certificates out of the extraCerts field into the local trust pool.

The proposal also includes the support of CMP over TLS as security transport mechanism to overcome the lack of protection at CMP protocol level on the extraCerts field, so the BTS knows that there are no intermediate proxies capable to insert non-authorized trust anchors.

Proposal 2: Use of ‘caPubs’ to transport self-signed root CA certificates

The CA which issues 3GPP operator base station certificates via CMPv2 sends all required self-signed Root CA certificates within the ‘ip’ (initialization response) or ‘kur’ (key update response) message as information element within the field ‘caPubs’ to the corresponding BTS. The BTS simply needs to copy those additional Root CA certificates out of the caPubs field into the local trust pool.

NOTE: Rest of sub-CA intermediate certificates can be transported in extraCerts field. The use of the ‘caPubs’ is limited for self-signed root CA certificates.

The use of CMP over TLS as security transport mechanism in this proposal is optional, as the protocol already supports integrity protection for ‘caPubs’.