3GPP TSG-SA3 Meeting #113 	S3-234558

# Chicago, US, 6 - 10 november 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Certificate bound access token in SBA OAuth framework

# Document for:	Discussion

Agenda Item:	6.2

# 1	Decision/action requested

It is presented for open discussion.

# 2	References

[1]	IETF RFC 8705 “OAuth 2.0 Mutual-TLS Client Authentication and Certificate-Bound Access Tokens

[2]	3GPP TS 33.501 “Security architecture and procedures for 5G system (Release 18)”

# 3	Rationale

IETF RFC 8705 [1] addresses the binding of the OAuth 2.0 access token with the TLS certificate issue, basically to mitigate the misuse of tokens, use of stolen tokens, or replay of access tokens by unauthorized parties. By binding access tokens to a specific client’s certificate, the risk of token theft and abuse is significantly reduced, as an attacker would need to compromise both the client’s private key and the token itself. It will also mitigate the token replay type of attacks. For the purpose of sender-constrained access tokens, the client is identified towards the resource server by the fingerprint of its public key. During processing of an access token request, the authorization server obtains the client's public key from the TLS stack and associates its fingerprint with the respective access tokens. The resource server in the same way obtains the public key from the TLS stack and compares its fingerprint with the fingerprint associated with the access token. The only constraint is the use of the same certificate for the client’s authentication with the authorization server and for the client’s authentication with the protected server.

However, [1] only addresses direct communication since it is relying on mutual TLS (mTLS), and it does not address the indirect communication cases (i.e., Model C and D) in 3GPP. SBA is using mTLS Client Authentication (as described in [1]) between NFc and SCP (model C/D), between SCP and NRF (model D), and between NFc and NRF (model C), but this does not hold when intermediate NRFs are used with hierarchical NRF (i.e., the final NRF does not have direct mTLS to the NFc). SBA does not use either certificate-bound access tokens as described in [1].

TS 33.501 [2] clause 13.4.1.1.2 says that: “… The NRF shall verify that the input parameters NF Instance ID and NF type as well as PLMN ID(s), if available, in the access token request match with the corresponding ones in the public key certificate of the NF Service Consumer or those in the NF profile of the NF Service Consumer. If the verification of the parameters in the access token request fails, the access token request is not further processed…”

TS 33.310 [3] clause 6.1.3c.2 says that: “… subjectAltName shall (in TLS client and server certificates) contain a URI-ID with the URI for the NF Instance ID as an URN; this URI-ID shall contain the nfInstanceID of the Network Function instance using the format of the NFInstanceId as described in clause 5.3.2 of TS 29.571 [57]...”

Observation 1 (Direct Communication): In case of 3GPP 5G Core (5GC) direct communication scenario one can assume that to certain extent the NRF may contribute to bind the certificate to the delivered access token after the verification of consumer’s parameters (if available) as indicated in TS 33.501 clause 13.4.1.1.2. I.e., the authentication of the NF consumer towards the NF producer to allow the latter to verify that the originator of the service request is the legitimate owner of the token, is implicit by the authorization made by the NRF when issuing the access token.

However, the access token itself does not have any explicit binding with the certificate used for mutual TLS authentication, nor indication on how, or if, the NRF completed the mutual authentication (or proof of possession binding) of the service client/NFc. Note that certificate-bound access tokens as specified in [1] require proof of possession through the use of the certificate .used to establish mutual TLS authentication.

Observation 2 (Indirect Communication): IETF RFC 8705, either the implicit authentication of the NF consumer specified in [2], are not directly applicable to indirect communication scenarios, where the communication between the NF consumer and the NF producer is done via SCP, since the mutual TLS authentication is performed between the NF and SCP.

Thus, the binding of authentication with authorization cannot be applied “as-is” in model C and D scenarios. Even if CCA token is present in the service request for indirect communication, as indicated in [2], clause 13.4.1.1., the NF producer may (optionally) verify that the subject claim in the access token matches the subject claim in the CCA token, and further the CCA token is to be checked against the certificate used for signing the CCA token, and *not* the mTLS client certificate (e.g. mTLS hop between SCP and NRF, NRF and NRF or SCP and NFp).

Conclusion: Overall certificate-bound access tokens leverage the level of security of the authorization framework as specified for 5GC, therefore it is proposed to study the applicability of this mechanism, mainly based on IETF RFC 8705 to 5GC, covering the use cases of indirect communication and use of CCA tokens.

# 4	Detailed proposal

Based on the argumentation exposed above two alternative proposals are presented:

1) Study the topic of certificate bound access token in SBA authorization framework as a mini-WID TEI19 in Rel-19.

2) Study the topic of certificate bound access token in SBA authorization framework as objective in the proposed SBA security enhancement in Rel-19