3GPP TSG-SA3 Meeting #113	S3-234590

# Chicago, US,  6th Nov 2023 - 10th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0189 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding SUPI/GPSI as an option in KAF request message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the lifetime of KAF expires and AF decides to refresh it, AF will send Naanf_AKMA_ApplicationKey_Get request message to AAnF with A-KID and AF_ID to request for KAF refresh. Then, the AAnF may request UDM to trigger a primary authentication to refresh KAKMA by sending Nudm_UECM_AuthTrigger Request message with UE’s SUPI. However, if the UE’s AKMA context is invalid or removed in the AAnF, the AAnF can not find UE’s SUPI based on received A-KID, which lead to a failure in KAF refresh. Such failure can be solved if AF could directly send SUPI/GPSI to AAnF. If SUPI/GPSI is received, AAnF has no need to check whether it has active AKMA context, and can directly use the received SUPI/GPSI to request for primary authentication. |  |  |  |  |  |  |  |  |
| AF |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | SUPI/GPSI is added as an option in KAF request message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KAF can not be refreshed if the AAnF is lack of AKMA context. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.3, 7.1.3, 7.1.5, 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* Start of 1st Change *************

6.2	Deriving AKMA Application Key for a specific AF

6.2.1	AAnF response with UE Identity

Figure 6.2-1 shows the procedure used by the AF to request application function specific AKMA keys from the AAnF, when the AF is located inside the operator's network.

Figure 6.2-1: KAF generation from KAKMA

Before communication between the UE and the AKMA AF can start, the UE and the AKMA AF need to know whether to use AKMA. This knowledge is implicit to the specific application on the UE and the AKMA AF or indicated by the AKMA AF to the UE (see clause 6.5).

1.	The UE shall generate the AKMA Anchor Key (KAKMA) and the A-KID from the KAUSF before initiating communication with an AKMA Application Function. When the UE initiates communication with the AKMA AF, it shall include the derived A-KID (see clause 6.1) in the Application Session Establishment Request message. The UE may derive KAF before sending the message or afterwards.

2.	If the AF does not have an active context associated with the A-KID, then the AF selects the AAnF as defined in clause 6.7, and sends a Naanf_AKMA_ApplicationKey_Get request to AAnF with the A-KID to request the KAF for the UE. The AF also includes its identity (AF_ID) in the request. If the KAF is expired, the AF should also include SUPI/GPSI of the UE in the request.

NOTE 1: the AF can decide whether to send SUPI or GPSI based on its local policy.

AF_ID consists of the FQDN of the AF and the Ua* security protocol identifier (see Annex A.4). The latter parameter identifies the security protocol that the AF will use with the UE.

The AAnF shall check whether the AAnF can provide the service to the AF based on the configured local policy or based on the authorization information available in the signalling (i.e., Oauth2.0 token). If it succeeds, the following procedures are executed. Otherwise, the AAnF shall reject the procedure.

If SUPI/GPSI is included in the request, the AAnF shall use the received SUPI/GPSI to request for home network triggered primary authentication by sending Nudm_UECM_AuthTrigger Request message to the UDM as defined in TS 33.501[2]. If the primary authentication is performed successfully, the AAnF shall continue with step 5.

NOTE 2: If the AAnF receives UE’s GPSI from the AF, the AAnF first sends a Nudm_SDM_Get Request to the UDM to fetch the SUPI of the UE, then uses SUPI to request for home network triggered primary authentication.

If SUPI/GPSI is not included in the request, the AAnF shall verify whether the subscriber is authorized to use AKMA based on the presence of the UE specific KAKMA key identified by the A-KID.

If KAKMA is present in AAnF, the AAnF shall continue with step 3.

If KAKMA is not present in the AAnF, the AAnF shall continue with step 6 with an error response.

3.	Once receiving the request from the AF, if the AAnF determines this specific AF needs GPSI, according to its local policy, the AAnF sends a Nudm_SDM_Get Request to the UDM to fetch the GPSI of the UE. If the specific AF does not need GPSI, the AAnF shall continue with step 5.

4.	The UDM responds with the GPSI of the UE. The AAnF shall store the received GPSI as part of UE’s AKMA context.

5.	The AAnF derives the AKMA Application Key (KAF) from KAKMA if it does not already have KAF.

The key derivation of KAF shall be performed as specified in Annex A.4.

6.	The AAnF sends Naanf_AKMA_ApplicationKey_Get response to the AF with SUPI, KAF and the KAF expiration time. Whether to send SUPI or GPSI is determined by AAnF based on the local policy.

7.	The AF sends the Application Session Establishment Response to the UE. If the information in step 6 indicates failure of AKMA key request, the AF shall reject the Application Session Establishment by including a failure cause. Afterwards, UE may trigger a new Application Session Establishment request with the latest A-KID to the AKMA AF.

************* End of 1st Change *************

************* Start of 2nd Change *************

6.3	AKMA Application Key request via NEF

Figure 6.3-1 shows the procedure used by the AF to request KAF from the AAnF via NEF, when the AF is located outside the operator's network.

Figure 6.3-1: AKMA Application Key request via NEF

1.	When the AF is about to request AKMA Application Key for the UE from the AAnF, e.g. when UE initiates application session establishment request as in clause 6.2.1, the AF discovers the HPLMN of the UE based on the A-KID and sends the request towards the AAnF via NEF service API. The request shall include the A-KID and the AF_ID and optionally UE Id not needed indication. If the KAF is expired, the AF should also include GPSI of the UE in the request.

NOTE:	In the case of architecture without CAPIF support, the AF is locally configured with the API termination points for the service. In the case of architecture with CAPIF support, the AF obtains the service API information from the CAPIF core function via the Availability of service APIs event notification or Service Discover Response as specified in TS 23.222 [5].

2.	If the AF is authorized by the NEF to request KAF, including the authorization after verification of the AF_ID in step 1, the NEF discovers and selects an AAnF as defined in clause 6.7.

3.	The NEF sends a Naanf_AKMA_ApplicationKey_Get request to the selected AAnF with the A-KID to request the KAF for the UE. If GPSI is received from AF, NEF shall also include GPSI in the request.

The AAnF shall process the request in the same way as specified in clause 6.2.1 with following changes:

If GPSI is included in the request, the AAnF first sends a Nudm_SDM_Get Request to the UDM to fetch the SUPI of the UE, then use SUPI to request for home network triggered primary authentication by sending Nudm_UECM_AuthTrigger Request message to the UDM as defined in TS 33.501[2]. If the primary authentication is performed successfully, the AAnF shall continue with step 4 in this clause.

If GPSI is not included in the request,

If KAKMA is present in AAnF, the AAnF shall continue with step 4 in this clause.

If KAKMA is not present in the AAnF, the AAnF shall continue with step 5 in this clause with an error response.

4.	The AAnF generates the KAF as specified in clause 6.2.1 and sends the response to the NEF with the KAF, the KAF expiration time (KAF exptime) and SUPI.

5.	The NEF forwards the response to the AF with the KAF, the KAF expiration time (KAF exptime) and optionally GPSI (external ID). Based on local policy, the NEF uses the Nudm_SubscriberDataManagement service which is specified in TS 29.503[11] to translate SUPI to GPSI (external ID) and optionally include GPSI (external ID) in the response. If UE Id not needed indication is received in the incoming request, the NEF shall not provide the GPSI (external ID) to AF. The NEF shall not send the SUPI to the AF.

************* End of 2nd Change *************

************* Start of 3rd Change *************

7.1.3	Naanf_AKMA_ApplicationKey_Get service operation

Service operation name: Naanf_AKMA_ApplicationKey_Get.

Description: The NF consumer requests AKMA Application Key and UE ID from the AAnF.

Input, Required: A-KID, AF_ID

Input, Optional: SUPI/GPSI

Output, Required: KAF, KAF expiration time and SUPI or GPSI.

Output, Optional: None.

NOTE: If the AKMA application key is expired, the NF consumer should include SUPI/GPSI in the request. Whether to choose SUPI or GPSI is based on the local policy of the consumer.

************* End of 3rd Change *************

************* Start of 4th Change *************

7.1.5	Naanf_AKMA_ApplicationKey_ AnonUser_Getservice operation

Service operation name: Naanf_AKMA_ApplicationKey_AnonUser_Get.

Description: The NF consumer requests only the AKMA Application Key from the AAnF. This service is for allowing anonymous user access to the AF based on A-KID (i.e., UE identification is not required at the AF). The A-KID functions as a temporary user identifier.

Input, Required: A-KID, AF_ID

Input, Optional: GPSI

Output, Required: KAF, KAF expiration time.

Output, Optional: None.

NOTE: If the AKMA application key is expired, the NF consumer should include GPSI in the request.

************* End of 4th Change *************

************* Start of 5th Change *************

7.3.2	Nnef_AKMA_ApplicationKey_Get service operation

Service operation name: Nnef_AKMA_ApplicationKey_Get.

Description: The NF consumer requests the NEF to provide AF related key material.

Input, Required: A-KID, AF_ID

Input, Optional: UEID not needed indication, GPSI

Output, Required: KAF, KAF expiration time.

Output, Optional: GPSI (external ID).

NOTE: If the AKMA application key is expired, the NF consumer should include GPSI in the request.

************* End of 5th Change *************