3GPP TSG-SA3 Meeting #113	S3-234666

# Chicago, USA, 6 – 11 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Security for Selective SCG Activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This draftCR aims to draft the potential of Selective SCG Activation. According to R2-2311607, RAN2 has agreed on the following principles for the Selective SCG activation procedure: - There's an element of "Rel-18 Conditional-Reconfiguration Information" that includes a List of Group-ID (potentially mapping to SN) and associated SK-counter values. - The UE includes the selected SK-counter value in the MN RRC Reconfiguration Complete message when it selects a new SK-counter value. There's an assumption that both the target SN and UE use the same key under typical circumstances, but RAN2 noted that the UE cannot identify a key mismatch. - If there's no key mismatch scenario, the network will use the received SK-counter value in the event of a failure. - For specific scenarios like Pcell-change/PSCell-change/SCG Release, if the SCPAC configuration is kept, the UE maintains the unused SK-counter values. - There's a distinction between SK-counter lists for SCPAC configurations and SK-counter values for CPAC configurations. - No specification changes are needed for UE behavior for the Scenario where a free SK-Counter is not available at the time of execution. This scenario can be avoided by NW configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the detailed security mechanism and procedures for selective SCG activation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The security protection for the selective SCG activation procedure is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.X (new), Annex A.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.10.2.X	Security mechanism and procedures for Selective SCG Activation

### 6.10.2.X.1		General

In selective SCG activation, the MN may provide one or several candidate SCG configuration(s) for one or multiple candidate SN(s) to the UE. The UE may select and execute precisely one of these conditional reconfigurations to change PSCell based on the measurement results on candidate target PSCells. The conditional reconfiguration for the selected PScell remains valid after the UE selects the target and executes the target cell access procedure. Thus, the UE can connect to the same SN several times without any further reconfiguration by the network.

### 6.10.2.X.2		Security context initialization for selective SCG activation

To prevent key-stream reuse when the UE switches back and forth to the same PSCell or the SN, the MN generates per candidate SN the sequence of multiple distinct SN Counter values during the selective SCG activation procedure. Each SN Counter value is unique, and the sequences are non-overlapping. These SN Counter values per SN shall be provided to the UE by the MN. The UE shall store these values.

For the side of each SN:

The MN derives the corresponding KSN as described in Annex A.16 based on the corresponding SN Counters values. The KSN keys are sent to the SN from the MN. The SN shall store the KSN keys in its security context.

### 6.10.2.X.3		Security mechanism for UE to access to target SN

When the UE accesses an SCG of a target SN, either for the first time or a subsequent time based on the same conditional reconfiguration, the UE shall select the first unused SN counter value in order among the stored ones associated with the target SN. Because all values are distinct, selecting the first unused one ensures that it is not previously used with the current KgNB. The UE then derives the corresponding KSN using the SN Counter as described in Annex A.16.

The SN chooses the first unused Ksn key for that UE to establish the security with the UE.

# * * * * Start of Second change * * * *

# A.16	Derivation of KSN

This input string is used when the MN and UE derive KSN. The following input parameters shall be used:

-	FC =0x79,

-	P0 = Value of the SN Counter as a non-negative integer,

-	L0 = length of the SN Counter value (i.e. 0x00 0x02).

The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB.

Editor’s note: This Annex A.16 is for Derivation of Ksn for DC, SA3 to analyse and decide whether a new dedicated FC value to be defined for selective SCG.

# * * * * End of changes * * * *