3GPP TSG-SA3 Meeting #113	S3-234678

# Chicago, USA, 6 - 11 November 2023	 (revision of S3ah-230014)

# Source:	Lenovo, Nokia, Nokia Shanghai Bell, Philips, Intel, Interdigital

# Title:	New SID on Security Enhancements for URSP in Roaming Scenarios

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	New SID on Security Enhancements for URSP in Roaming Scenarios

Acronym:	FS_URSP_Sec

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A | N/A | {optional free text} |

Dependency on non-3GPP (draft) specification:

# 3	Justification

In roaming scenarios, URSP rules are sent from HPLMN to UE, via VPLMN, without any end-to-end security protection. Those URSP rules contain policy information for Access Network Discovery & Selection, UE Route Selection, V2X, ProSe, Ranging/Sidelink Positioning and Aircraft-to-Everything (A2X). The scope of the URSP rules is expanding with all these features and the new functionalities.

SA2 asked SA3 in an LS in S3‑223166 on the protection of the URSP rules from HPLMN and SA3 briefly discussed the issue and replied in S3‑223911 to SA2 a simple “no” on the need for protection without further details.

SA3 never analyzed the possible attacks and frauds that could be carried out in such roaming cases. For example, it is possible for VPLMN to change the URSP rules received from HPLMN to achieve an advantage e.g. replacing NSSAIs, downgrade service performance to save network resources etc. Since the URSP rules are sent via the V-PCF in the VPLMN, it is easy to modify the rules at one place only to achieve an effect in the whole VPLMN network for all inbound roamers.

The trust in the VPLMN is limited in 5GS, the security keys are bound to the serving network name, the SUPI is only sent after successful authentication and the authentication endpoint remains in the HPLMN for full home control. Further crucial information sent from the HPLMN to the UE in the VPLMN with UPU or SoR are protected due to that limited trust.

# 4	Objective

The objective is to study

- WT1: whether there is sensitive information in the URSP rules that can be misused in the serving network

- WT2: how to protect the sensitive information, if any, in the URSP rules towards the UE

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT1 | 2 |  | No | -- |
| WT2 | 1 | 1 | No | Depends on outcome of WT1 |

Total TU estimates for the study phase:   3

Total TU estimates for the normative phase:   1

Total TU estimates: 4

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | N/A | Security Enhancements for URSP in Roaming Scenarios | SA#104 | SA#105 | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A | N/A | N/A |  |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

System architecture aspects, i.e. URSP procedures, will be covered by SA2

# 9	Supporting Individual Members

Supporting IM name

Lenovo

Nokia

Nokia Shanghai Bell

Philips

Intel

Interdigital