3GPP TSG-SA3 Meeting #113 	S3-234715

# Chicago, US, 6 -10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0008 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 SL positioning - Updates on UE discovery procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the discovery of ProSe capable UEs for the Ranging/SL positioning, it is specified to reuse the security mechanisms of restricted 5G ProSe Direct Discovery. However, since the ProSe service code (i.e., ProSe Restricted Code for model A, ProSe Query Code and ProSe Response Code for model B) are not defined for Ranging/SL Positioning UE discovery, the restricted 5G ProSe Direct Discovery procedure cannot be used as currently specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The User Info ID is included instead of RPAUID in the Discovery Request message. The ProSe Restricted Code, ProSe Query Code and ProSe Response Code are not included in the Discovery Response message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Discovery security materials cannot be provisioned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF 1st CHANGE*******

### 6.2.3	Security procedures for ProSe capable UEs

The security mechanisms for both models of restricted 5G ProSe Direct Discovery defined in clause 6.1.3.2 of TS 33.503 [6] are reused for ProSe capable UEs to provide protection for Ranging/SL positioning UE discovery with the following changes:

- The SLPKMF rather than 5G DDNMF is used to provision discovery security materials for Ranging/SL positioning UE discovery.

- The User Info ID is included instead of the RPAUID in the Discovery Request message.

- The ProSe Restricted Code, ProSe Query Code and ProSe Response Code are not included in the Discovery Response message.

************ END OF 1st CHANGE*******