3GPP TSG-SA3 Meeting #113	S3-234722

# Chicago, United States, 6th Nov 2023 - 10th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1844 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Validation of the parameters in the access token request in interconnect scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR specifies security aspects of the 5G Service Based Architecture Phase 2, as concluded by the Study on enhanced Security Aspects of the 5G Service Based Architecture. The parameters included in the access token request sent by the OAuth 2.0 client (NF Service consumer) are validated by the NRF. In the current specification it is unclear which parameters should be validated by which NRF in interconnect scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In interconnect scenarios, the NRF in consumer PLMN authenticates the NF Service Consumer and shall verify the input parameters in the access token request as described for the vNRF under Step 1 in clause 13.4.1.2.2. And the producer PLMN NRF checks whether the NF Service Consumer is authorized to access the requested service(s) as described for the hNRF under Step 1 in clause 13.4.1.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The OAuth 2.0 client may claim to have certain attributes that the OAuth 2.0 client does not have. Consequently, if NRF in consumer PLMN does not verify the input paramters, the OAuth 2.0 client may be able to trick the NRF into giving an access token to the OAuth 2.0 client that the OAuth 2.0 client is not entitled to obtain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

#### 13.4.1.1A	Service access authorization in interconnect scenarios

In the inter-PLMN interconnect scenario, OAuth 2.0 roles are as follows:

a.	The NF Service Consumer's Network Repository Function (cNRF) shall be the OAuth 2.0 Authorization server for the PLMN of the NF Service Consumer (cPLMN) and authenticates the NF Service Consumer.

b.	The NF Service Producer's Network Repository Function (pNRF) shall be OAuth 2.0 Authorization server for the PLMN of the NF Service Producer (pPLMN) and generates the access token.

c.	The NF Service Consumer in the cPLMN shall be the OAuth 2.0 client.

d.	The NF Service Producer in the pPLMN shall be the OAuth 2.0 resource server.

As an example of the inter-PLMN interconnect use case, service access authorization in the roaming scenario where the service consumer NF is located in the visiting PLMN and the service producer NF is located in the home PLMN is specified in clause 13.4.1.2.

An NF Service Consumer shall send its access token requests to the NRF in the consumer PLMN where it is registered as OAuth 2.0 client. The NRF in consumer PLMN authenticates the NF Service Consumer, and shall verify the input parameters in the access token request as described for the vNRF under Step 1 in clause 13.4.1.2.2. If the verification of the parameters in the access token request fails, the access token request is not further processed. After successful authentication and verification of the input parameters, the NRF in the consumer PLMN forwards the access token request to the NRF in the producer PLMN as described for the vNRF and hNRF under Step 1 in clause 13.4.1.2.2. The NRF in the producer PLMN checks whether the NF Service Consumer is authorized to access the requested service(s) as described for the hNRF under Step 1 in clause 13.4.1.2.2.

*** END CHANGES ***