3GPP TSG-SA3 Meeting #113 	S3-234736

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0011 | rev | <Rev#> | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Privacy profile for Ranging SL positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According the discussion in S3-234735, this paper proposes to solve the UE privacy file for Ranging/SL positioning service exposure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Annex to define UE privacy profile for Ranging/SL positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing privacy file for Ranging /SL Positioning service exposure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5, Annex XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGE ****

### 6.3.5	Procedures for authorization of AF/5GC NF for Ranging/SL positioning service exposure

For the authorization of the AF or 5GC NF for Ranging/SL Positioning service exposure, the SL-MT-LR procedure specified in TS 23.273 [3] is taken as the baseline. The authorization shall be performed towards all the n UEs (n ≥ 2), i.e. UE1, UE2, ..., UEn in the request message. If all of the UEs don't grant permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request from the AF/5GC NF.

When receiving the Ranging/SL Positioning service request from the AF/5GC NF, the GMLC interacts with the UDM to check the UE privacy profile for Ranging/SL Positioning service as specified in Annex XX.

NOTE:	The details of the UE privacy profile for Ranging/SL positioning services needs to be align with SA2.

The GMLC interacts with the AMF to request the ranging result of UEs, which may include an indication of a privacy related action. If the indicator of privacy check related action indicates that the UE must either be notified or notified with privacy verification, a notification invoke message is sent to the UE if the signalling connection established. However, if the Ranging/SL Positioning service is disallowed by the UE, or signalling connection establishment fails and UE notification (including UE notification with privacy verification) is required, the AMF shall provide failure response to the GMLC.

**** NEXT CHANGE ****

Annex XX (normative):
UE privacy profile for Ranging/SL Positioning service

UE privacy profile for Ranging/SL Positioning service is used to control whether Ranging/SL positioning service exposure is permitted.

For each UE subscriber, privacy preferences are stored in UE privacy profile for Ranging/SL Positioning service as part of UE subscription data in the UDM and queried from the UDM by another NF such as GMLC. The UDM may also store the UE privacy profile in the UDR.

For UE Positioning service assisted by Sidelink Positioning over 5GC-MT-LR service, the UE LCS privacy profile defined in clause 5.4.2 of TS 23.273 [3] is used.

The UE privacy profile data for Ranging/SL Positioning service is defined in table XX-1.

Table XX-1: Ranging/SL Positioning privacy profile data stored in the UDM for a UE Subscriber

| Privacy Profile Data Type | Presence | UDM data |
| --- | --- | --- |
| Ranging/SL Positioning Privacy Indication | M O | Indication of one of the following mutually exclusive global settings: - Ranging/SL Positioning service is disallowed - Ranging/SL Positioning service is allowed (default) Time period when the Ranging/SL Positioning Privacy Indication is valid |

**** END OF CHANGE ****