3GPP TSG-SA3 Meeting #113 	S3-234784

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0044 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identification of RNAA token |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPPY |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This contribution aims to solve the RNAA differentiation part in the following Editor’s Note used in two places: Editor's Note: further details of the token are left for stage 3, this includes how to differentiate RNAA and legacy tokens A specific claim for the resource owner ID is needed for this differentiation, because in some use cases the token includes GPSI (e.g., tokens in edge computing), also in the future there can be some other new cases where tokens include GPSI. Since the API invoker ID is already in the legacy token claims, it is not necessary to restate the mandatory requirement on having the API invoker ID in the token. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A specific claim for resource owner ID is used to differentiate RNAA and legacy tokens. Redundant mandatory requirement statement of having resource owner ID in the token is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There can be ambiguity for AEF to identify whether the received token is a RNAA or a legacy token. Redundant statements will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.1, 6.5.3.2, 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ******* START OF CHANGES *******

#### 6.5.3.1	General

RNAA shall use token-based authorization using OAuth 2.0 framework with the following roles:

-	The API invoker has the role of the OAuth 2.0 client.

-	The CCF has the role of the OAuth 2.0 authorization server, i.e., providing the access token used for RNAA.

-	The AEF has the role of the resource server.

The access tokens used for RNAA can contain the resource owner identity claim and other claims.The resource owner can be the user of the UE or the owner of the subscription depending on the use case and regulations. The present document does not specify the resource owner, but the resource owner ID is specified as the GPSI of the corresponding UE if the resource is related to a UE.

The access token shall include the resource owner ID in addition to the token profile in Annex C. A specific claim for the resource owner ID shall be used to differentiate RNAA and legacy tokens. GPSI is used as identifier for the resource owner. The token issuer ID is the CCF ID.  The API invoker ID binds the token to the API invoker. To avoid privacy issues, GPSI needs to be different from MSISDN, SUPI etc.

Editor's Note: The details of access tokens used for RNAA need to be aligned with stage 3 (e.g., claim versus scope).

AEF shall do the authorization check of the API invocation request. AEF checks the request against the token, including the resource owner identity. As the token includes resource owner identity, there is no need for additional UE authentication in API invocation. Moreover, the token should be able to restrict the API invoker to a specific resource (e.g., location, QoS, PDN connectivity status).

For Oauth flows involving redirection, authentication between CCF/AUF and UE should be performed after API Invoker redirects the UE to CCF/AUF.

In case of an external AF (i.e., not the application on the UE) being the API invoker, for mutual authentication of API invoker AF and API exposing function, the authentication methods of clause 6.4 and 6.5.2 are  reused.

For authorization, the following flows can be used:

-	Client credential flow (according to RFC 6749 [4]),

-	Authorization code flow (according to RFC 6749 [4]), or

-	Authorization code flow with PKCE (according to RFC 7636 [11]).

Editor's Note: How to choose the flow is left to stage 3.

CCF shall give service authorization which subscribers or users can use RNAA.

NOTE: In this specification, only a UE accessing its own resources is considered if the API invoker is on a UE.

## ******* NEXT CHANGE *******

#### 6.5.3.2	Authorization using oauth client credential flow

If client credential flow is used for authorization of the API invoker by the AEF, the procedures in RFC 6749 [4] shall be followed with the following profile:

-	The access token request message may include the resource owner ID.

NOTE 1: If the API invoker is on a UE, the CCF obtains its GPSI during authentication.

Editor’s note: the mapping of API Invoker ID and GPSI is left for stage 3.

-	The CCF shall check whether the API invoker is entitled to consume the API and allowed to access the resources of the resource owner, by using authorization information available in the CCF.

-	If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. If the API invoker is an AF not on a UE, the check is omitted.

Editor's Note: Further details of the token are left for stage 3.

NOTE 2: How to get the authorization from the resource owner and store it in the CCF is out of scope of the present document.

## ******* NEXT CHANGE *******

#### 6.5.3.3	Authorization using authorization code (optional PKCE) flow

If authorization code flow, optionally with PKCE, is used by the AEF for authorization of the API invoker, the procedures in RFC 6749 [4] and optionally RFC 7636 [11] shall be followed, with the following profile:

-	The authorization token and/or authorization request may include the resource owner ID.

Editor’s Note: Whether and how the token and/or authorization request can include resource owner ID is left to stage 3.

NOTE: If the API invoker is on a UE, the CCF obtains its GPSI during authentication.

Editor's note: the mapping of API Invoker ID and GPSI is left for stage 3.

-	The resource owner dynamically allows the API invoker to access the resource owner's resources as described in RFC 6749 [4] and optionally RFC 7636 [11].

-	If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. The access token shall contain the resource owner ID (i.e. GPSI) and the API invoker ID. If the API invoker is an AF not on a UE, the check is omitted.

Editor's Note: further details of the token are left for stage 3.

## ******* END OF CHANGES *******