3GPP TSG-SA3 Meeting #113 	S3-234785

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0045 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Optimizations for accessing own resources |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPPY |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the case of that the API invoker (e.g., the AF or the UE) is accessing its own resources, it is not necessary to include the resource owner ID in the token request. This will remove the requirement of having to know the resource owner ID, which would be very valuable in some use cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An optimization is added to remove the requirement of including the resource owner ID in the token request if it is about accessing own resources. Also regarding this optimization, a clarification on the check of allowing access the own resources of the UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The API invoker will have to know the resource owner ID even if it wants to access its own resources. For example, the application running on a UE will have to know the GPSI of the UE to be able to consume the northbound API to access the UE’s own resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ******* START OF CHANGE *******

#### 6.5.3.2	Authorization using oauth client credential flow

If client credential flow is used for authorization of the API invoker by the AEF, the procedures in RFC 6749 [4] shall be followed with the following profile:

-	The access token request message may include the resource owner ID or an indication that the API invoker requests access to its own resources.

NOTE 1: If the API invoker is on a UE, the CCF obtains its GPSI during authentication.

If there is no resource owner ID in the token request nor an indication about accessing own resources, the CCF can determine the resource owner ID as the API invoker ID (i.e., the GPSI of the UE) which has been obtained during the authentication of the API invoker by the CCF. It is assumed that authorization information about this usage if available in the CCF.

Editor’s note: the mapping of API Invoker ID and GPSI is left for stage 3.

-	The CCF shall check whether the API invoker is entitled to consume the API and allowed to access the resources of the resource owner, by using authorization information available in the CCF.

-	If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. If the request includes a GPSI and the API invoker is a UE and the GPSI is different from the API invoker ID, then the CCF shall reject the request. If the API invoker is an AF not on a UE, the check is omitted.

Editor's Note: Further details of the token are left for stage 3, this includes how to differentiate RNAA and legacy tokens

NOTE 2: How to get the authorization from the resource owner and store it in the CCF is out of scope of the present document.

## ******* END OF CHANGE *******