3GPP TSG-SA3 Meeting #113 	S3-234787

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0047 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the scope of the Rel-18 RNAA specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPPY |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are some points that are not specified and left to out of scope. These points need to be clearly stated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Out of scope points are stated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.1, 6.5.3.2, 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ******* START OF CHANGES *******

#### 6.5.3.1	General

The RNAA specification in the present document only applies to the case that the resources to be accessed are related to UE subscription.

RNAA shall use token-based authorization using OAuth 2.0 framework with the following roles:

-	The API invoker has the role of the OAuth 2.0 client.

-	The CCF has the role of the OAuth 2.0 authorization server, i.e., providing the access token used for RNAA.

-	The AEF has the role of the resource server.

The access tokens used for RNAA can contain the resource owner identity claim and other claims.

The resource owner can be the user of the UE or the owner of the subscription depending on the use case and regulations. The present document does not specify the resource owner, but the resource owner ID is specified as the GPSI of the corresponding UE if the resource is related to a UE.

The access token shall include the resource owner ID and the API invoker ID. GPSI is used as identifier for the resource owner . The token issuer ID is the CCF ID.  The API invoker ID binds the token to the API invoker. To avoid privacy issues, GPSI needs to be different from MSISDN, SUPI etc.

Editor's Note: The details of access tokens used for RNAA need to be aligned with stage 3 (e.g., claim versus scope).

AEF shall do the authorization check of the API invocation request. AEF checks the request against the token, including the resource owner identity. As the token includes resource owner identity, there is no need for additional UE authentication in API invocation. Moreover, the token should be able to restrict the API invoker to a specific resource (e.g., location, QoS, PDN connectivity status).

For Oauth flows involving redirection, authentication between CCF/AUF and UE should be performed after API Invoker redirects the UE to CCF/AUF.

In case of an external AF (i.e., not the application on the UE) being the API invoker, for mutual authentication of API invoker AF and API exposing function, the authentication methods of clause 6.4 and 6.5.2 are  reused.

For authorization, the following flows can be used:

-	Client credential flow (according to RFC 6749 [4]),

-	Authorization code flow (according to RFC 6749 [4]), or

-	Authorization code flow with PKCE (according to RFC 7636 [11]).

Editor's Note: How to choose the flow is left to stage 3.

CCF shall give service authorization which subscribers or users can use RNAA. For this authorization control, the CCF can check whether the resource owner is entitled to use RNAA for the requested API, by using configured information in the CCF. How this information is configured or provisioned to the CCF is not specified in the present document.

NOTE: In this specification, only a UE accessing its own resources is considered if the API invoker is on a UE.

## ******* NEXT CHANGE *******

#### 6.5.3.2	Authorization using oauth client credential flow

If client credential flow is used for authorization of the API invoker by the AEF, the procedures in RFC 6749 [4] shall be followed with the following profile:

-	The access token request message may include the resource owner ID.

NOTE 1: If the API invoker is on a UE, the CCF obtains the UE's GPSI during authentication by using a mechanism not specified in the present document.

Editor’s note: the mapping of API Invoker ID and GPSI is left for stage 3.

-	The CCF shall check whether the API invoker is entitled to consume the API and allowed to access the resources of the resource owner, by using authorization information available in the CCF.

-	If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. If the API invoker is an AF not on a UE, the check is omitted.

Editor's Note: Further details of the token are left for stage 3, this includes how to differentiate RNAA and legacy tokens

NOTE 2: How to get the authorization from the resource owner and store it in the CCF is out of scope of the present document.

## ******* NEXT CHANGE *******

#### 6.5.3.3	Authorization using authorization code (optional PKCE) flow

If authorization code flow, optionally with PKCE, is used by the AEF for authorization of the API invoker, the procedures in RFC 6749 [4] and optionally RFC 7636 [11] shall be followed, with the following profile:

-	The authorization token and/or authorization request may include the resource owner ID.

Editor’s Note: Whether and how the token and/or authorization request can include resource owner ID is left to stage 3.

NOTE: If the API invoker is on a UE, the CCF obtains the UE's GPSI during authentication by using a mechanism not specified in the present document.

Editor's note: the mapping of API Invoker ID and GPSI is left for stage 3.

-	The resource owner dynamically allows the API invoker to access the resource owner's resources as described in RFC 6749 [4] and optionally RFC 7636 [11].

-	If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. The access token shall contain the resource owner ID (i.e. GPSI) and the API invoker ID. If the API invoker is an AF not on a UE, the check is omitted.

Editor's Note: further details of the token are left for stage 3, this includes how to differentiate RNAA and legacy tokens

The authentication of the resource owner by the CCF is not specified in the present document.

## ******* END OF CHANGE *******