3GPP TSG-SA3 Meeting #113 	S3-234788

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.558 | CR | 0016 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the GPSI verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | GPSI verification by the EES needs correction because of the following two points: - The existing text say that "the EES shall retrieve the GPSI from the core network no matter whether the EEC sends the GPSI to the ECS", but EES cannot execute the condition because it is not aware whether the EEC has sent the GPSI to the ECS. - If the GPSI is verified by the ECS (i.e., if it is included in the access token issued by the ECS), then the EES should not be forced to learn the GPSI from the core network because there is no privacy threat and this interaction with the core network is unnecessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The condition about GPSI verification is corrected and "should" is used to recommend getting the GPSI from the core network. - A note is added to state that if the GPSI is in the token, then the EES can use it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unclear specification - Forcing the EES to do an unnecessary operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ******* START of CHANGE *******

## 6.3	Authentication and authorization between EEC and EES

Authentication between EEC and EES shall be done during the execution of the TLS handshake protocol. Server side certificate-based TLS authentication shall be supported. Details of the authentication method (e.g., TLS certificates, usage of AKMA [11] or GBA [12] as methods to arrange the PSK for TLS) are out of scope of the present document.

NOTE 1: 	Usage of application layer solutions for EEC authentication is left to implementation.

NOTE 2:	If only server side certificate-based TLS authentication is performed, it is left to implementation on which information within a service procedure and services will be provided by the EES.

The authentication method negotiation mechanism shall re-use the existing TLS v1.3 negotiation. UE may receive the supported authentication method of the EES optionally as part of the EES configuration information. Details of the EES configuration information are specified in TS 23.558 [5]. If the UE has the information about the authentication method supported by the EES, then the EEC/UE may use this information for the authentication method negotiation.

NOTE 3:	Further optimization regarding having prior knowledge about the capability, such as UE storing the selected algorithm from the past negotiation results, is left to EEC/UE implementation.

If the GPSI is required, the EES shall retrieve the GPSI from the core network no matter whether the EEC sends the GPSI to the EES.

NOTE 4: If the EES identifies a mismatch between the GPSI received from the EEC and the GPSI received from the network, the decision and action to be taken by the EES for such mismatch cases are left to implementation.

NOTE X: If the EES receives the GPSI in an access token from the EEC, the EES can use the GPSI without needing to retrieve the GPSI from the core network.

For authorization of EEC by the EES, the EEC shall send the OAuth 2.0 [15] access token, if received from the ECS, to the EES. The token profile is specified in clause 6.2 of the present document. If the EES requires access token for authorization, then the EES shall authorize the EEC by using the token. Otherwise, the EES shall authorize the EEC by its local authorization policy.

After successful authentication and authorization, the EES shall process the request and sends the service response back to the EEC.

## ******* END of CHANGE *******