3GPP TSG-SA3 Meeting #113 	S3-234789

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.558 | CR | 0017 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on EDGE-10 interface to cover the ECS-ER security |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Security of EDGE-10 reference point has been addressed but the reference point is mentioned only for the communication between V-ECS and H-ECS. In TS23.558, this reference point is also used for ECS-ER procedures. To address the security requirements of ECS-ER procedures, extending the security scope of EDGE-10 reference point is necessary. - EDGE-10 security is missing in clause 5.1.2 - ECS-ER is missing in clause 6.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Requirements are added for the communication between ECS and ECS-ER and between ECS-ERs are added and a new subclause is introduced for the security of these communications. - EDGE-10 securiy is added in clause 5.1.2 - ECS-ER is added in clause 6.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be missing points about EDGE-10 security. The security of the ECS-ER procedures will be underspecified. Current security specification for EC federation will be underspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.2, 6.1, 6.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ******* START of CHANGES *******

### 5.1.1	Authentication and authorization

Authentication and Authorization between Edge Enabler Client (EEC) and Edge Configuration Server (ECS): Edge Configuration Server (ECS) shall be able to provide mutual authentication with Edge Enabler Client (EEC) over EDGE-4 Interface. ECS shall determine whether EEC is authorized to access ECS's services.

Authentication and Authorization between EEC and EES: Edge Enabler Server (EES) shall provide mutual authentication with EEC over EDGE-1 Interface. EES shall determine whether EEC is authorized to access EES's services.

Authentication and Authorization between Edge Enabler Server (EES) and ECS: ECS shall provide mutual authentication with EES over EDGE-6 Interface. ECS shall determine whether EES is authorized to access ECS's services.

Authentication and Authorization between EESs: EES shall provide mutual authentication with another EES over EDGE-9 Interface. EES shall determine whether peer EES is authorized to access EES's services.

Authentication and Authorization in EES capability exposure to EAS: EES shall provide mutual authentication with EAS over EDGE-3 Interface. EES shall determine whether EAS is authorized to access EES's services and expose EEC Capabilities. The Edge application architecture shall support EASs to obtain the user's authorization to access sensitive information (e.g. user's location).

NOTE1: The corresponding security requirements defined in TS 23.558 [5] is AR-5.2.6.2-a/b/d/e/f/g.

Authentication and Authorization between Application Client (AC) and EEC: EEC should provide mutual authentication with the Application Client over EDGE-5 interface, and the EEC should determine whether Application client is authorized to access EEC’s service.

Authentication and Authorization between V-ECS and H-ECS：V-ECS shall provide mutual authentication with H-ECS over EDGE-10 Interface. V-ECS shall determine whether H-ECS is authorized to access V-ECS's services.

Authentication and Authorization between ECS and ECS-ER: ECS-ER shall provide mutual authentication with ECS over EDGE-10 interface. ECS-ER shall determine whether ECS is authorized to access ECS-ER's services.

Authentication and Authorization between ECS-ERs: ECS-ERs shall provide mutual authentication over EDGE-10 Interface. ECS-ER shall determine whether other ECS-ERs are authorized to access ECS-ER's services.

## ******* NEXT CHANGE *******

### 5.1.2	Interface security

Confidentiality, integrity, and replay protection shall be supported on the EDGE-1-4 and EDGE 6-10 interfaces.

NOTE 1: The interfaces are defined in the Figure 6.2.4 of TS 23.558 [5]. The corresponding security requirement defined in TS 23.558 [5] is AR-5.2.6.2-c.

NOTE 2: The security requirement of EDGE 5 is out of the scope of this specification, since its details are out of the scope of this release of this specification, according to TS 23.558 [5].

The privacy requirements AR-5.2.6.2-h defined in TS 23.558 [5] are implicitly supported, since all the interfaces will be confidentiality and integrity protected.

## ******* NEXT CHANGE *******

## 6.1	Security for the EDGE interfaces

For the interfaces (EDGE-1/4), the EEC, EES and ECS shall support and use HTTP/2 with "https" URIs as specified in RFC 9113 [19] and RFC 9110 [20]. In addition, the TLS profile shall be compliant with the profile given in clause 6.2 of TS 33.210 [2] .

For the interfaces EDGE-2/7/8,

-	If the NEF APIs are selected, security aspects of Network Exposure Function including the protection of NEF-AF interface and support of CAPIF defined in TS 33.501 clause 12 [2] shall be reused, i.e.,  use of TLS.

-	If the SCEF APIs are selected, the Security procedures for reference point SCEF-SCS/AS defined in TS 33.187 clause 5.5 [3] can be reused here, i.e., use of TLS.

For the interfaces (EDGE-3/6/9/10), the EAS, EES, ECS, and ECS-ER shall support and use HTTP/2 with "https" URIs as specified in RFC 9113 [19] and RFC 9110 [20]. In addition, the TLS profile shall be compliant with the profile given in clause 6.2 of TS 33.210 [2].

## ******* NEXT CHANGE *******

## 6.X	Authentication and authorization between ECS and ECS-ER and between ECS-ERs

The ECS and ECS-ERs are provisioned with credentials (e.g., certificate, shared keys/secrets) for mutual authentication. The mutual authentication between ECS and ECS-ER and between ECS-ERs shall be done based on the preconfigured credentials. The ECS-ER shall authorize the ECS and other ECS-ERs based on local authorization policy.

## ******* END of CHANGES *******