3GPP TSG-SA3 Meeting #113	S3-234828

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1878 | rev | - | Current version: | 17.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [IAB][Rel-17] IAB inter-CU topology adaptation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Intel, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When the IAB-MT is migrated from a source IAB-donor-CU to a target IAB-donor-CU, the migrating IAB-node becomes a boundary IAB-node since its IAB-DU retains F1AP with the source IAB-donor-CU while its IAB-MT obtains RRC connectivity with the target IAB-donor-CU detailed in TS 38.401 [78]. In this case, it is required to specify the security procedure to re-establish the F1 security in case if the PSK is computed dynamically. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If dynamic PSK computation for IKEv2 PSK authentication is used,then the IKEv2 between migrating IAB-node and the source IAB-donor-CU is performed using stored KIAB as PSK. For IPsec transport mode, the gNB-DU ID provided in the IKE_AUTH message is used to identify the PSK(s). Further, a NOTE is included to address the scenario when there are more than one old/new IP addresses, by performaing the IKEv2 procedures according to the TNL address index, as to have identical mapping between the old TNL address with the new TNL address in the source/Initial IAB-donor-CU and in the migrating/descendant/recovery IAB-node. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in preforming security procedure for IAB inter-CU topology adaptation procedure may make migrating IAB-nodes out-of-service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | M.3.3.2, M.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************Start of Changes*********************

### M.3.3.2	Security mechanisms for the F1 interface

The F1 interface connects the IAB-node (gNB-DU) to the IAB-donor-CU. It consists of the F1-C for control plane and the F1-U for the user plane.

F1 security for IAB is established using the security mechanisms for the F1 interface as specified in clause 9.8.2 of the present document, with IAB node taking the role of gNB-DU and IAB-donor-CU taking the role of gNB-CU.

In addition to the security mechanisms specified in clause 9.8.2 of the present document for the F1 interface, the IKEv2 Pre-shared Secret Key (PSK) authentication shall be supported. When IKEv2 performs a PSK authentication, in the IKE_AUTH request message, the IAB node shall set the ID type to ID_KEY-ID and set its value to PSK ID.

NOTE 1:	The PSK and PSK ID (for IKEv2 PSK authentication) can be preconfigured at the IAB node and IAB donor. Pre-configuration of the PSK(s) is out of the scope of the present document.

Additionally, to support a flexible plug and play of IAB-node and IAB-donor without a pre-configuration of the PSK(s), dynamic PSK computation for IKEv2 PSK authentication may also be supported. When dynamic PSK is used, the IAB-node and the IAB-donor shall calculate the PSK (KIAB) as specified in the Annex A.23 of this document. The IAB-donor shall uniquely identify the IAB-node’s security context (KgNB) using the IAB-node DU IP address. The IAB-donor shall use KIAB as PSK for IKEv2 between IAB-node and the IAB-donor. KIAB is stored in the IAB-node and in the IAB-donor. This key KIAB and the IPsec SA cryptographic keys are taken into use with the establishment of IPsec Security Association (SA) between the IAB-node and the IAB-donor. KIAB remains valid as long as the IAB-node is connected to the IAB-donor or until the IAB-node is re-authenticated. The KIAB is associated with the gNB-DU ID and IAB-node DU IP address in the IAB-donor. In the IKE_AUTH request message, the IAB node shall set the ID type to ID_KEY-ID and set its value to gNB-DU ID. In case of CP-UP separation of IAB-donor-CU (IAB-donor-CU contains IAB-donor-CU-CP and IAB-donor-CU-UP that use different IP address) then, IAB-donor-CU-CP and IAB-node DU shall generate KIAB-CU-CP and KIAB-CU-UP as specified in the Annex A.23 of this document. The key KIAB-CU-CP shall be used for establishment of secure F1 interface between the IAB-node DU and IAB-donor-CU-CP. The IAB-donor-CU-CP shall provide KIAB-CU-UP to the IAB-donor-CU-UP via E1 interface and KIAB-CU-UP shall be used for establishment of secure F1 interface between the IAB-node DU and IAB-donor-CU-UP.

NOTE 2:	KIAB is used as the PSK for IKEv2 authentication, the interface between the IAB-donor-CU and the SEG to provision the key KIAB in the SEG is implementation specific and out of the scope of the present document.

******************Next Change*********************

# M.5	IAB inter-CU topology adaptation and backhaul RLF recovery procedure

In case of the inter-CU migration as specified in TS 38.401 [78], the IAB-MT is migrated from a source IAB-donor-CU to a target IAB-donor-CU. The migrating IAB-node becomes a boundary IAB-node since its IAB-DU retains F1AP with the source IAB-donor-CU after its IAB-MT obtains RRC connectivity with the target IAB-donor-CU (c.f. TS 38.401 [78]).

In case IPsec tunnel mode is used for F1 interface protection, the migrating/descendant/Recovery IAB-node may use MOBIKE (IETF RFC 4555 [111]) to migrate the IPsec tunnel to the new IP outer addresses as specified in TS 38.401 [78].

In case IPsec transport mode is used for F1 interface protection and dynamic PSK computation for IKEv2 PSK authentication is used, then new IKEv2 between migrating/descendant/Recovery IAB-node and the source/Initial IAB-donor-CU may be performed using the KIAB associated with the old IP address as the PSK for the corresponding new IP address.

NOTE x:	In case of multiple TNL addresses, the identical mapping between the old TNL address with the new TNL address between the IAB-node and the IAB-donor-CU is left to the implementation. An implementation option is to perform the IKEv2 procedures according to the TNL address index.

*******************End of Changes*********************