3GPP TSG-SA3 Meeting #113 	S3-234831

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1880 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security for Selective SCG Activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-10-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This draftCR aims to narrow down the procedure for selective sCG activation agreed in last SA3 meeting. Based on RAN2 progress following ENs are resolved and the procedure is updated accordingly: Editor’s Note: Further collaborate with RAN2 to decide the specific message for the MN to signal the SN Counter values per SN to UE, and to signal KSN key(s) to SN, respectively. Editor’s Note: Further collaborate with RAN2 to decide which option (1 or 2 or 3) to use for SN. Editor’s note: This Annex A.16 is for Derivation of Ksn for DC, SA3 to analyse and decide whether a new dedicated FC value to be defined for selective SCG. - As updated in RAN2 running CR R2-2309830 for conditional SN change, MN provides the CPAC configurations or subsequest CPAC configurations to the UE in RRC reconfiguration message. Hence, it is proposed to include the SN counter(s) in the CPAC configurations for subsequent evaluation. - Further, as in Option 1 and 3 of S3-234164, it is not a good security principle to derive bulk of KSN(s) based on the generated SN counters. In addition, there may be a chance that some SN counters remain unused. To avoid such wastage of resource of generation of keys and storage in prior it is suggested to go with Option 2. - Clause A.16 can be generalized for both Dual connectivity and selective SCG. Further we can inline security procedure for selective SCG activation as follows: - The MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC. - The UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. The UE changes the SN Counter upon every SN change. - UE sends the SN counter used to the MN in RRC reconfiguration complete message. - MN derives the KSN using the received SN counter and provides the key to the SN in SN RRC reconfiguration complete message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the detailed security mechanism and procedures for selective SCG activation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The security protection for selective SCG activation procedure is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.X (new clause), Annex A.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.10.2.X	Security mechanism and procedures for Selective SCG Activation

### 6.10.2.X.1		General

In the scenario of selective SCG activation, the MN may provide one or several candidate SCG configuration(s) for one or multiple candidate SN(s) to the UE. The UE may select and execute precisely one of these conditional reconfigurations in order to change PSCell based on the measurement results on candidate target PSCells. The conditional reconfiguration for the selected PScell remains valid after the UE selected the target and executed the target cell access procedure. Thus the UE is able to connect to the same SN several times without any further reconfiguration by the network.

### 6.10.2.X.2		Security context initialisation for selective SCG activation

In order to prevent key-stream reuse when the UE switches back and forth to the same PSCell or the SN, the MN assigns per candidate SN the sequence of multiple distinct SN Counter values maintained for dual connectivity during the selective SCG activation procedure. Each SN Counter value is unique, and the sequences are non-overlapping. These SN Counter values per SN shall be provided to the UE by the MN. The MN sends RRC Connection Reconfiguration Request containing the configurations for each SN to the UE. The UE shall store these received SN Counter values. The MN shall maintain the largest unassigned SN Counter value in the sequence for KSN calculation for DC as described in subclause 6.10.3.1 of this document or for further assignment for selective SCG activation as described in this subclause.

When a new AS root key, KNG-RAN, in the associated 5G AS security context is established and the SN Counter is set to ‘0’ as described in subclause 6.10.3.1, the MN assigns new sequence of multiple distinct SN Counter values per candidate SN to the UE. The UE shall delete the stored SN Counter values and store the received new SN Counter values.

### 6.10.2.X.3		Security mechanism for UE to access to target SN

When the UE accesses an SCG of a target SN, either for the first time or for a subsequent time based on the same conditional reconfiguration, the UE shall select the first unused SN counter value in order among the stored ones associated with the target SN. Because all values are distinct, selecting the first unused one ensures that it is not previously used together with the current KgNB. The UE then derives the corresponding KSN using the SN Counter as described in Annex A.16 of this document. The UE shall send the value of the SN counter (used to generate the KSN) to the MN in RRC Connection Reconfiguration complete message. Upon receiving the RRC Connection Reconfiguration complete message from the UE, the MN calculates KSN using the received SN Counter value. The calculated KSN is sent to the target SN from the MN in SN Reconfiguration complete message.

# * * * * Start of Second change * * * *

# A.16	Derivation of KSN

This input string is used when the MN and UE derive KSN. The following input parameters shall be used:

-	FC =0x79,

-	P0 = Value of the SN Counter as a non-negative integer,

-	L0 = length of the SN Counter value (i.e. 0x00 0x02).

The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB.

# * * * * End of changes * * * *