3GPP TSG-SA3 Meeting #113 	S3-234843

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | 0143 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE-to-UE Relay Communication with integrated discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G ProSe PC5 Communication with integrated Discovery is specified in TS 23.304 [2]. This paper proposes to add security for integrated discovery specified in clause 6.7.3 of TS 23.304 [2]. The security for PC5 link establishment in 5G ProSe PC5 Communication with integrated Discovery between an End UE (Source/Target UE) and a UE-to-UE relay can be established with network assistance based on the procedure as specified in clause 6.6.3.1 with the following differences and modifications: - When the second hop PC5 link security between the 5G ProSe Layer-3 UE-to-UE Relay and the Target End UE has been successfully completed, then the 5G ProSe UE-to-UE Relay triggers the first hop PC5 link security establishment with the Source End UE. - Only the 5G ProSe Layer-3 UE-to-UE relays supporting the indicated security method (i.e., with network assistance) and in 3GPP coverage responds to the broadcasted Direct Communication Request message and broadcast a Direct Communication Request message in its proximity. Otherwise, the 5G ProSe Layer-3 UE-to-UE relays ignore the received broadcasted Direct Communication Request message. - The 5G ProSe Target UE receives the broadcasted Direct Communication Request messages and selects one 5G ProSe Layer-3 UE-to-UE relay to proceed PC5 link security establishment procedure, considering the indicated security method (i.e., with network assistance) in the DCR. - The broadcasted DCR message is protected by reusing the protection method defined in clause 6.3.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G ProSe PC5 Communication with integrated Discovery is specified in TS 23.304 [2]. This paper proposes to add security for integrated discovery specified in clause 6.7.3 of TS 23.304 [2]. Note that the heading 6.6.5 was deleted from the skeleton of the draft CR before draft CR was converted to a CR to TS 33.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security for 5G ProSe UE-to-UE Relay Communication with integrated Discovery with network assistance is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 6.6.x, (new) 6.6.x.1, (new) 6.6.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGE ****

### 6.6.x	Security for 5G ProSe UE-to-UE Relay Communication with integrated Discovery

#### 6.6.x.1	General

The security for PC5 link establishment in 5G ProSe PC5 Communication with integrated Discovery between an End UE (Source/Target UE) and a UE-to-UE relay can be established with network assistance and without network assistance.

#### 6.6.x.2	Security of 5G ProSe PC5 Communication with integrated Discovery for 5G ProSe Layer-3 UE-to-UE Relay with network assistance

The security for PC5 link establishment in 5G ProSe PC5 Communication with integrated Discovery between an End UE (Source/Target UE) and a UE-to-UE relay is established with network assistance as specified in clause 6.6.3.1 with the following modifications:

- When the second hop PC5 link security between the 5G ProSe Layer-3 UE-to-UE Relay and the Target End UE has been successfully completed, then the 5G ProSe UE-to-UE Relay triggers the first hop PC5 link security establishment with the Source End UE.

- Only the 5G ProSe Layer-3 UE-to-UE relays supporting the indicated security method (i.e., with network assistance) and in 3GPP coverage respond to the broadcasted Direct Communication Request message and broadcast a Direct Communication Request message in its proximity. Otherwise, the 5G ProSe Layer-3 UE-to-UE relays ignore the received broadcasted Direct Communication Request message.

- The 5G ProSe Target UE receives the broadcasted Direct Communication Request messages and selects one 5G ProSe Layer-3 UE-to-UE relay to proceed PC5 link security establishment procedure, considering the indicated security method (i.e., with network assistance) in the DCR.

The broadcasted Direct Communication Request messages are sent protected (integrity protected and confidentiality protected) by reusing the protection method defined in clause 6.3.5 between the End UE’s (Source/Target UE) and the UE-to-UE relay.

**** END OF CHANGE ****