3GPP TSG-SA3 Meeting #113 	S3-234881

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0018 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Common Security in Clause 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As specified in clause 4.2.1.1, the SLPKMF has the similar functionalities as those of 5G PKMF specified in TS 33.503, which supports the reference point Npc10 with the UDM for UE authorization. For SLPKMF, there are the same requirements on the interaction with the UDM for UE authorization during security procedures for discovery, unicast direct communication, and broadcast/groupcast communication. This requires a service-based inteface (Npc10*) between the SLPKMF and UDM. Both Npc9* (as defined in clause 4.2.2) and Npc10* support service based architecture and are service-based interfaces. The security for these two reference points needs to be added as a common security for Ranging/SL positioning services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a subclause in clause 5 for security of service-based interfaces used in Ranging/SL positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security for service-based interfaces used in Ranging/SL positioning is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, new clause 5.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

5	Common security

5.1	General

This clause describes the security requirements and procedures commonly applied to different features of Ranging/SL positioning services, including Ranging/SL positioning discovery, Ranging/SL positioning communication with unicast mode, broadcast/groupcast mode, etc.

5.2	Security for PC8* interface

5.2.1	General

The security requirements on PC8* interface between ProSe capable UE and SLPKMF are derived based on the definition of the SLPKMF described in clause 4.2.1.1 and the definition of PC8* interface described in clause 4.2.2.

5.2.2	Security requirements

The 5G System shall support mutual authentication between the ProSe capable UE and the SLPKMF.

The 5G System shall support integrity protection for the transmission between the ProSe capable UE and the SLPKMF.

The 5G System shall support confidentiality protection for the transmission between the ProSe capable UE and the SLPKMF.

The 5G System shall support anti-replay protection for the transmission between the ProSe capable UE and the SLPKMF.

5.2.3	Security procedures for PC8* using GBA

When using GBA for the security procedures on PC8* interface, the use of either TLS v1.2 or TLS v. 1.3 as described in clause 5.3.3.2 of TS 33.303 [9] applies with the following changes:

-	The SLPKMF takes the role of ProSe function.

-	Confidentiality protection shall be enabled.

5.2.4	Security procedures for PC8* using AKMA

When using AKMA for the security procedures on PC8* interface, the specification in clause B.1.3.2 of TS 33.535 [10] applies with the following changes:

-	The SLPKMF takes the role of AF.

-	Confidentiality protection shall be enabled.

5.x	Security of service-based interfaces used in Ranging/SL Positioning

Npc9* and Npc10* defined in clause 4.2.2 are realized by corresponding NF service-based interfaces. Security procedures specified in clause 13 of TS 33.501 [11] apply to these interfaces.

*************** End of the Change ****************