3GPP TSG-SA3 Meeting #113 	S3-234900

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0030 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the authorization for UEs belonging to different PLMNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As defined in TS 23.586, inter-PLMN operation is supported for Ranging services, e.g. the Target UE is subscribed in PLMN A and the Reference UE is subscribed in PLMN B. According to the authorization procedure for Ranging service exposure defined in clause 6.3.5, all the involved UEs need to be authorized for protecting the privacy of the UEs, i.e. the privacy profiles of the UEs in potentially different PLMNs need to be checked. This requires the GMLC in one PLMN which receives the Ranging service request shall be able to request the GMLC in another PLMN to get the authorization result of the UE in another PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the NL3 interface between GMLCs used for UE privacy authorization for Ranging service exposure. - Added the description for the authorization of UEs belongling to different PLMNs. - Added the GMLC service operations for UE privacy authorization to support inter-PLMN scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The GMLC in one PLMN cannot check the authorization of all the involved UEs in case the UEs belong to different PLMNs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 6.3.5, new clause 7.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

4.2.2	Reference points

In addition to the reference points specified in clause 4.2 of TS 23.586 [2], the following reference points are added or enhanced for supporting Ranging/SL positioning security architecture:

NL3:	The reference point between the GMLCs in different PLMNs. It is used to transport the UE authorization result for authorization of Ranging/SL Positioning service exposure.

NL6:	The reference point between the UDM and the GMLC. It is used to transport the UE privacy profile to GMLC for authorization of Ranging/SL Positioning service exposure.

Editor's Note: Whether NL6 can be used to resolve UE identity between application layer ID and SUPI for protecting UE privacy is to be aligned with SA2.

PC8*:	The reference point between the UE and the SLPKMF, which relies on 5GC user plane for transport (i.e. an "over IP" reference point). It is used to transport security materials to UEs for Ranging/SL Positioning discovery and communication.

Npc9*:	The reference point between the SLPKMFs of the UEs subscribed in different PLMNs. It is used to transport security materials between SLPKMFs to support Ranging/SL Positioning services in inter-PLMN scenarios.

*************** Start of the 2nd Change ****************

6.3.5	Procedures for authorization of AF/5GC NF for Ranging/SL positioning service exposure

For the authorization of the AF or 5GC NF for Ranging/SL Positioning service exposure, the SL-MT-LR procedure specified in TS 23.273 [3] is taken as the baseline. The authorization shall be performed towards all the n UEs (n ≥ 2), i.e. UE1, UE2, ..., UEn in the request message. If the involved UEs belong to different PLMNs, NL3 is used for the GMLC receiving the service request to obtain the authorization result of UE(s) belonging to other PLMNs. If all of the UEs don't grant permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request from the AF/5GC NF.

NOTE:	Once receiving the Ranging/SL positioning service request containing UE Application Layer ID, the GMLC receiving the service request determines the address of the GMLC in abother PLMN based on local configuration or by the NRF query.

When receiving the Ranging/SL Positioning service request from the AF/5GC NF, the GMLC interacts with the UDM to check the UE privacy profile. The UE LCS Privacy Profile defined in clause 5.4.2 of TS 23.273 [3] is taken as the baseline for the UE privacy profile for Ranging/SL positioning services.

NOTE:	The details of the UE privacy profile for Ranging/SL positioning services needs to be align with SA2.

The GMLC interacts with the AMF to request the ranging result of UEs, which may include an indication of a privacy related action. If the indicator of privacy check related action indicates that the UE must either be notified or notified with privacy verification, a notification invoke message is sent to the UE if the signalling connection established. However, if the Ranging/SL Positioning service is disallowed by the UE, or signalling connection establishment fails and UE notification (including UE notification with privacy verification) is required, the AMF shall provide failure response to the GMLC.

*************** Start of the 3rd Change ****************

7.x	GMLC services

7.x.1	General

The following table illustrates the GMLC Services and Service Operations.

Table 7.x.1-1: List of GMLC Services

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ngmlc_Authorization | Ngmlc_Authorization_Get | Request/Response | GMLC |

7.x.2	Ngmlc_Authorization service

7.x.2.1	Ngmlc_Authorization_Get service operation

Service operation name: Ngmlc_Authorization_Get

Description: The NF consumer obtains the authorization result of the UE from another PLMN for Ranging/SL positioning service exposure.

Input, Required: UE Application Layer ID(s), Ranging/SL Positioning Application Identifier.

Input, Optional: None.

Output, Required: Success/Failure result.

Output, Optional: None.

*************** End of the Change ****************