3GPP TSG-SA3 Meeting #113 	S3-234902

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0032 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the UE Ranging/SL Positioning privacy profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the reply LS S3-234480, SA2 has confirmed that the content of UE Ranging/SL Positioning privacy profile should be developed by SA3. As currently specified in clause 6.3.5, UE Ranging/SL Positioning privacy profile can take the LCS privacy profile as the baseline. But the differences between UE LCS privacy profile and UE Ranging/SL positioning privacy profile are not clarified, which need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the differences between UE LCS privacy profile and UE Ranging/SL positioning privacy profile in clause 6.3.5. - Added the content of UE Ranging/SL Positioning privacy profile as a new Annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of details of the UE Ranging/SL Positioning privacy profile, without which the GMLC is not able to perform authorization for Ranging service exposure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5, new Annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

6.3.5	Procedures for authorization of AF/5GC NF for Ranging/SL positioning service exposure

For the authorization of the AF or 5GC NF for Ranging/SL Positioning service exposure, the SL-MT-LR procedure specified in TS 23.273 [3] is taken as the baseline. The authorization shall be performed towards all the n UEs (n ≥ 2), i.e. UE1, UE2, ..., UEn in the request message. If all of the UEs don't grant permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request from the AF/5GC NF.

When receiving the Ranging/SL Positioning service request from the AF/5GC NF, the GMLC interacts with the UDM to check the UE Ranging/SL Positioning privacy profile. The UE LCS Privacy Profile defined in clause 5.4.2 of TS 23.273 [3] is taken as the baseline for the UE Ranging/SL Positioning privacy profile with the following modifications:

- 	UE Ranging/SL Positioning privacy profile is part of subscription data for UEs subscribed to use Ranging/SL positioning services.

-	UE Ranging/SL Positioning privacy profile is used to indicate whether Ranging/SL positioning service exposure to 5GC NF, AF, LCS Client and SL Positioning Client UE is allowed or disallowed.

-	The list of external LCS client in UE LCS privacy profile is replaced by the list of AF/SL Positioning Client UE.

-	The list of service type in UE LCS privacy profile is replaced by the list of Ranging/SL Positioning Application Identifier.

-	The list of LCS client in UE LCS privacy profile is replaced by the list of 5GC NF/LCS client.

The details of the UE privacy profile for Ranging/SL positioning services are defined in Annex X.

The GMLC interacts with the AMF to request the ranging result of UEs, which may include an indication of a privacy related action. If the indicator of privacy check related action indicates that the UE must either be notified or notified with privacy verification, a notification invoke message is sent to the UE if the signalling connection established. However, if the Ranging/SL Positioning service is disallowed by the UE, or signalling connection establishment fails and UE notification (including UE notification with privacy verification) is required, the AMF shall provide failure response to the GMLC.

*************** Start of the 2nd Change ****************

Annex X (normative):
UE Ranging/SL Positioning privacy profile

The UE Ranging/SL Positioning privacy profile data is defined in table B-1 containing data for the privacy classes for which location of the UE is permitted.

Table X-1: Ranging/SL Positioning privacy profile data stored in the UDM for a UE Subscriber

| Privacy Profile Data Type | Presence | UDM data |
| --- | --- | --- |
| Ranging/SL Positioning Privacy Indication | M O | Indication of one of the following mutually exclusive global settings: - Ranging/SL Positioning services is disallowed - Ranging/SL Positioning services is allowed (default) Time period when the Ranging/SL Positioning Privacy Indication is valid |
| Call/session Unrelated Class | M O O O O O O O O O O O O O | For any AF or SL Positioning Client UE not in the AF/SL Positioning Client UE list or otherwise identified for the Call/session Unrelated Class, the following data may be present: - One of the following mutually exclusive options: - Ranging/SL positioning result not allowed (default case) - Ranging/SL positioning result allowed with notification - Ranging/SL positioning result with notification and privacy verification; location allowed if no response - Ranging/SL positioning result with notification and privacy verification; location restricted if no response - Time period when Ranging/SL positioning is allowed - Geographical area where Ranging/SL positioning is allowed - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC AF/SL Positionng Client UE list: a list of zero or more AFs/SL Positioning Client UEs with the following data for each entry: - One of the following mutually exclusive options: - Ranging/SL positioning result allowed without notification (default case) - Ranging/SL positioning result allowed with notification - Ranging/SL positioning result with notification and privacy verification; location allowed if no response - Ranging/SL positioning result with notification and privacy verification; location restricted if no response - Time period when Ranging/SL positioning is allowed - Geographical area where Ranging/SL positioning is allowed Ranging/SL Positioning Application Identifier list: a list of one or more Ranging/SL Positioning Application Identifiers for which the Ranging/SL Positioning application is allowed to range/locate the particular UE. The following data may be present for each Ranging/SL Positioning Application Identifier in the list: - One of the following mutually exclusive options: - Ranging/SL positioning result allowed without notification (default case) - Ranging/SL positioning result allowed with notification - Ranging/SL positioning result with notification and privacy verification; location allowed if no response - Ranging/SL positioning result with notification and privacy verification; location restricted if no response - Time period when Ranging/SL positioning is allowed - Geographical area where Ranging/SL positioning is allowed - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC |
| PLMN Operator Class | O | NF/LCS client list: a list of one or more generic classes of NF/LCS client that are allowed to locate the particular UE. The following classes are distinguished: - LCS client broadcasting location related information - O&M LCS client in the HPLMN - O&M LCS client in the VPLMN - LCS client recording anonymous location information - LCS Client supporting a bearer service, teleservice or supplementary service to the target UE - NWDAF in the HPLMN (when the UE is currently being served by the HPLMN) - NWDAF in the VPLMN |
| Event report expected area | O | Presents a geographical area generated by UE, which is used by GMLC to determine event report allowed area for the UE |
| GMLC address list | O | Addresses of GMLC located in local network(s) which are allowed to be used for the UE Ranging/SL positioning |

*************** End of the Changes ****************