3GPP TSG-SA3 Meeting #112 	S3-234904

# Chicago, US, 6 - 10 November 2023

Title:	[Draft] Reply LS on Retrieving keys for decryption of protected IEs for U2N relay

Response to:	LS S3-234453/C1-234362 on Retrieving keys for decryption of protected IEs for U2N relay from CT1

Release:	Rel 17

Work Item:	5G_ProSe

Source:	Xiaomi (to be SA3)

To:	CT1

Cc:	-

Contact person:	Jouy Shang

shangzhengyi@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA3 would like to thank CT1 for their LS on Retrieving keys for decryption of protected IEs for U2N relay. SA3 would like to provide the following feedback.

How does the 5G ProSe UE-to-network relay UE retrieve the corresponding discovery security parameters for decryption of the relay service code and the UP-PRUK ID/CP-PRUK ID in the PROSE DIRECT LINK ESTABLISHMENT REQUEST message?

Answer:	SA3 has identified the same issue in the Direct Discovery also (besides UE-to-Network Relay discovery), i.e. it is not specified how the 5G ProSe UE retrieves the corresponding discovery security parameters for decryption of the PC5 discovery messages?

For this common issue, SA3 agrees that the 5G ProSe UE can retrieve the discovery security parameters based on the Destination Layer-2 ID of the received message, assuming the Destination Layer-2 ID is associated with one ProSe service or one ProSe Relay service. If the Destination Layer-2 ID is associated with multiple ProSe services or ProSe Relay services, the 5G ProSe UE needs to try multiple sets of associated discovery security parameters.

Is there any additional security mechanism needed to be defined by SA3?

Answer:	No additional security mechanism will be defined by SA3. SA3 kindly requests CT1 to specify the corresponding mechanism in stage 3.

2	Actions

To CT1

ACTION: SA3 kindly asks CT1 to take the above information into account.

3	Dates of next TSG SA WG 3 meetings

SA3#114	22 -26 January 2024	Athens