3GPP TSG-SA3 Meeting #113 	S3-234908

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0052 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve EN related to authorization flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPPY |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 33.122, client credentials flow, authorization code flow and authorization code flow with PKCE were introduced to support RNAA scenarios. Two of these flows (i.e. authorization code flow and authorization code flow with PKCE) are new to the CAPIF system. Before the API invoker starts to obtain authorization from the resource owner, the authorization flow should be chosen for the API invoker. However, there is a lack of authorization flow selection mechanism in RNAA scenarios. And the Editor's Note "How to choose the flow is left to stage 3" in clause 6.5.3.1 of TS 33.122 is captured to highlight the issue. Since SA3 is responsible for designing security mechanisms, this contribution tries to resolve the Editor's Note by reusing the security method negotiation mechanism defined in clause 6.3.1.2 of TS 33.122. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The security method negotiation mechanism defined in clause 6.3.1.2 of TS 33.122 is used for RNAA with the following modification. In step 4, the CAPIF core function shall send Security Method Response message to the API invoker, indicating the selected security method (e.g., authorization code flow, authorization code flow with PKCE) for each AEF. -The following Editor's Note is resolved. Editor's Note: How to choose the flow is left to stage 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Editor's Note related to authorization flow is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.2, 6.5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************Start of the 1st Change ****************

6.3.1.2	Security method negotiation

The API invoker and the CAPIF core function shall negotiate a security method that shall be used by the API invoker and the API exposing function for CAPIF-2e interface authentication and protection. After successful mutual authentication on CAPIF-1e interface, based on the API invoker's subscribed service APIs, access scenarios (whether the API invoker access the AEF prior to service API invocation or upon the service API invocation) and AEF capabilities, the CAPIF core function shall choose the security method and sends the chosen security methods along with the information required for authentication of the API invoker at the AEF to the API invoker. The information may include the validity time of the CAPIF-2e credentials. This is depicted in figure 6.3.1-1.

Pre-conditions:

1.	The API invoker is onboarded with the CAPIF core function.

Figure 6.3.1-1: Selection of security method to be used in CAPIF-2/2e reference point

1.	Mutual authentication based on client and server certificates shall be established using TLS between the API invoker and the CAPIF core function. The client certificate that was provided to the API invoker as the result of successful onboarding is used based on the description in subclause 6.1 of the present document.

2.	The API invoker may send CAPIF-2/2e security capability information to the CAPIF core function in the Security Method Request message, indicating the list of security methods that the API invoker supports over CAPIF-2/2e reference point for each AEF.

3.	The CAPIF core function shall select a security method to be used over CAPIF-2/2e reference point for each requested AEF, taking into account the information from the API invoker in step 2, access scenarios and AEF capabilities.

4.	The CAPIF core function shall send Security Method Response message to the API invoker, indicating the selected security method (e.g., authorization code flow, authorization code flow with PKCE) for each AEF, any security information related to the security method. The API invoker shall use this method in the subsequent communication establishment with the API exposing function over CAPIF-2/2e reference point, as described in subclause 6.5 of the present document.

***************Start of the 2nd Change ****************

6.5.3.1	General

RNAA shall use token-based authorization using OAuth 2.0 framework with the following roles:

-	The API invoker has the role of the OAuth 2.0 client.

-	The CCF has the role of the OAuth 2.0 authorization server, i.e., providing the access token used for RNAA.

-	The AEF has the role of the resource server.

The access tokens used for RNAA can contain the resource owner identity claim and other claims.

The resource owner can be the user of the UE or the owner of the subscription depending on the use case and regulations. The present document does not specify the resource owner, but the resource owner ID is specified as the GPSI of the corresponding UE if the resource is related to a UE.

The access token shall include the resource owner ID and the API invoker ID. GPSI is used as identifier for the resource owner. The token issuer ID is the CCF ID.  The API invoker ID binds the token to the API invoker. To avoid privacy issues, GPSI needs to be different from MSISDN, SUPI etc.

Editor's Note: The details of access tokens used for RNAA need to be aligned with stage 3 (e.g., claim versus scope).

AEF shall do the authorization check of the API invocation request. AEF checks the request against the token, including the resource owner identity. As the token includes resource owner identity, there is no need for additional UE authentication in API invocation. Moreover, the token should be able to restrict the API invoker to a specific resource (e.g., location, QoS, PDN connectivity status).

For Oauth flows involving redirection, authentication between CCF/AUF and UE should be performed after API Invoker redirects the UE to CCF/AUF.

In case of an external AF (i.e., not the application on the UE) being the API invoker, for mutual authentication of API invoker AF and API exposing function, the authentication methods of clause 6.4 and 6.5.2 are  reused.

For authorization, the following flows may be used:

-	Client credential flow (according to RFC 6749 [4]),

-	Authorization code flow (according to RFC 6749 [4]), or

-	Authorization code flow with PKCE (according to RFC 7636 [11]).

Security method negotiation mechanism defined in clause 6.5.3.1 is used to choose the authorization flow.

CCF shall give service authorization which subscribers or users can use RNAA.

NOTE: In this specification, only a UE accessing its own resources is considered if the API invoker is on a UE.

***************End of the Changes ****************