3GPP TSG-SA3 Meeting #113 	S3-234961

# Chicago, US, 6 - 10 november 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0056 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving stage 2 editor's notes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPPY |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Resolution of stage 2 editor's notes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.2 EN can be deleted, because the figure is already aligned with 23.222 Clause 6.5.3.1 EN is unclear what is the task for stage 3. Accordingly, it is clarified that the flow to be used depends on application requirements, while stage 3 needs to define how to communicate which flow to be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved editor's notes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******** First Change ********

## 5.2	Functional security model supporting RNAA

Figure 5.2-1 shows the functional security architecture of CAPIF when RNAA is supported.  The authorization function itself is a part of the CCF. The Oauth client and the CCF shall communicate using https.

Figure 5.2-1: CAPIF supporting RNAA functional security model

******** Next Change ********

#### 6.5.3.1	General

RNAA shall use token-based authorization using OAuth 2.0 framework with the following roles:

-	The API invoker has the role of the OAuth 2.0 client.

-	The CCF has the role of the OAuth 2.0 authorization server, i.e., providing the access token used for RNAA.

-	The AEF has the role of the resource server.

The access tokens used for RNAA can contain the resource owner identity claim and other claims.

The resource owner can be the user of the UE or the owner of the subscription depending on the use case and regulations. The present document does not specify the resource owner, but the resource owner ID is specified as the GPSI of the corresponding UE if the resource is related to a UE.

The access token shall include the resource owner ID and the API invoker ID. GPSI is used as identifier for the resource owner. The token issuer ID is the CCF ID.  The API invoker ID binds the token to the API invoker. To avoid privacy issues, GPSI needs to be different from MSISDN, SUPI etc.

Editor's Note: The details of access tokens used for RNAA need to be aligned with stage 3 (e.g., claim versus scope).

AEF shall do the authorization check of the API invocation request. AEF checks the request against the token, including the resource owner identity. As the token includes resource owner identity, there is no need for additional UE authentication in API invocation. Moreover, the token should be able to restrict the API invoker to a specific resource (e.g., location, QoS, PDN connectivity status).

For Oauth flows involving redirection, authentication between CCF/AUF and UE should be performed after API Invoker redirects the UE to CCF/AUF.

In case of an external AF (i.e., not the application on the UE) being the API invoker, for mutual authentication of API invoker AF and API exposing function, the authentication methods of clause 6.4 and 6.5.2 are  reused.

Depending on application requirements, for authorization, the following flows can be used:

-	Client credential flow (according to RFC 6749 [4]),

-	Authorization code flow (according to RFC 6749 [4]), or

-	Authorization code flow with PKCE (according to RFC 7636 [11]).

Editor's Note: communicate which flow is to be used is left to stage 3.

CCF shall give service authorization which subscribers or users can use RNAA.

NOTE: In this specification, only a UE accessing its own resources is considered if the API invoker is on a UE.

******* End of changes **********