3GPP TSG-SA3 Meeting #113	S3-235053

Chicago, US, 6 -10 November 2023

Title:	Reply LS on Handling of SOR counter and the UE parameter update counter if stored in NVM

Response to:	S3-234445/C1-232696 LS on Handling of SOR counter and the UE parameter update counter if stored in NVM

Release:	Rel-18

Work Item:	TEI18

Source:	SA3

To:	CT1

Cc:	CT4, CT6

Contact person:	Ivy Guo

ivy_guo@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-235052

# 1	Overall description

SA3 would like to thank CT1 for their LS C1-232696/S3-234445 on handling the SOR counter and the UE parameter update counter if stored in NVM.

SA3 would like CT1 to know that SA3 discussed the LS, acknowledged the issue and agreed on the attached CR.

SA3 would like also to bring to CT1’s attention the following observations.

When configuring Rel-16 and beyond USIM, if operator requests to have Service n°123 "available" in the EFUST, then the recommendation is to also have Service n°133 "available" in the EFUST. So, the problem described in LS from CT1 should not take place on the field for USIM from Rel-16 and beyond.

For Rel-15 USIM on the field with Service n°123 available, a way to solve the issue is to update by OTA (Over- The-Air) the USIM in order to make "available" the Service n°133 with associated counters in the file EF5GAUTHKEYS. This update of the Rel-15 USIM solves the issue without any impact on the ME. A USIM can be updated by OTA if it has been issued with the adequate OTA capabilities.

# 2	Actions

To CT1:

ACTION: 	SA3 kindly asks CT1 to take the content of the LS into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#114                                                                 26 February-1st March 2024                                                   Athens, GR

SA3#115                                                                20 -24 May 2024                                                                      Korea, KR