3GPP TSG-SA3 Meeting #113 	S3-235029

# Chicago, US, 6 - 10 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0022 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add privacy handing for Ranging/SL positioning service exposure through 5GC CP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA2 requested SA3 to provide feedback on Q4 asked in S3-232329, as follows: Q4: For Ranging/SL positioning service exposure to the SL positioning client UE, either through PC5 or through the 5GC network, privacy aspects of exposing the location of the Target UE/SL Reference UE may need to be studied by SA3. In authorization procedure for Ranging/SL positioning service exposure through 5GC control plane defined in clause 6.3.6.2, only the authorization of the SL positioning Client UE by the AMF is specified. There is no specification on how the privacy of the Target UE/SL Reference UEs is handled for lcoation exposure. According to the procedure for Ranging/SL positioning service exposure through 5GC control plane defined in clause 6.7.1.2.3 of TS 23.586, the AMF shall forward the SL-MO-LR request from the Client UE to the GMLC, which triggers the SL-MT-LR procedure. Then the GMLC is able to reuse the procedure of privacy check for the to-be-measured UEs as defined in clause 6.3.5 for SL-MT-LR procedure. Therefore, it is proposed to add the privacy profile check for the to-be-measured UEs when Ranging/SL positioning service is exposed through 5GC control plane, by reusing the procedure defined in clause 6.3.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added privacy profile check for the to-be-measured UEs in clause 6.3.6.2, by reusing the procedure defined in clause 6.3.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Privacy of the to-be-measured UEs is not protected when Ranging/SL positioning service is exposed to the Client UE through 5GC control plane . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

6.3.6.2	Authorization procedure for Ranging/SL positioning service exposure through 5GC control plane

For Ranging/SL Positioning service exposure through the network via control plane (i.e. clause 6.7.1.2.3 of TS 23.586 [2]), the MO-LR authorization as specified in TS 23.273 [3] is reused for the authorization of the SL positioning Client UE.

If the Client UE is not authorized, the Ranging/SL Positioning service request shall be rejected.

To preserve the privacy of the Target/Reference UEs, the privacy profiles of the Target/Reference UEs shall be checked. Upon receiving Ngmlc_Location_ProvideRanging_Request from the AMF (i.e. step 3 in clause 6.7.1.2.3 of TS 23.586 [2]), the GMLC shall perform the privacy check for the Target/Reference UEs as defined in clause 6.3.5 during the SL-MT-LR procedure (i.e. step 4 in clause 6.7.1.2.3 of TS 23.586 [2]).

*************** End of the Change ****************