3GPP TSG-SA3 Meeting #113 	S3-235110

Chicago, US, 6 - 10 November 2023

Title:	LS on Model Sharing With MTLF

Response to:

Work Item:	eNA_Ph3_SEC

Source:	SA3

To:	3GPP SA2

Cc:

Contact person:	Christine Jost

Christine.jost@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA2 TS 23.288 clause 5.3 describes that an MTLF can request the trained Model from another MTLF acting as FL server, either based on its local configuration or triggered by the request from another NWDAF containing AnLF. Excerpts below:

"If the NWDAF containing MTLF determines to train an ML model based on local configuration and the FL mechanism is required, but the NWDAF containing MTLF can't act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML model as described in clause 6.2C.2.2. The FL server NWDAF may determine to initiate FL procedure before providing the ML model.

If the ML model training is triggered by the request from NWDAF containing AnLF, the NWDAF containing MTLF determines the FL mechanism is required but it can not act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML model as described in clause 6.2C.2.2. The Subscription endpoint address of the NWDAF containing AnLF is provided in the request message sent to the FL server NWDAF. The FL server NWDAF may determine to initiate FL procedure before providing the ML model. The FL server NWDAF sends the ML model information to the notification endpoint (e.g. the NWDAF containing AnLF) after the ML model training success.

NOTE 2:	How to authorize an MTLF to request ML models on behalf of an AnLF to another MTLF (e.g., FL server NWDAF) is up to SA WG3."

SA3 has discussed the use cases of the above and has acknowledged the security issue.

SA3 has not agreed on a security solution.

# 2	Actions

To 3GPP SA2

ACTION: 	SA3 kindly asks SA2 to take the above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#115	26 Feb -01 March 2024	Athens (Greece)

SA3#116	20 -24 May 2024		South Korea