- 3GPP TSG-SA3 Meeting #115AdHoc-e	draft_S3-241181-r1

- Electronic meeting, online, 15 - 19 April 2024	revision of S3-24yyyy

- Source:	vivo, OPPO, Xiaomi, Apple

- Title:	New key issue on algorithm negotiation for 256-bit algorithms

Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

Approve the pCR on new key issue on algorithm negotiation for 256-bit algorithms.

# 2	References

# 3		Rationale

This contribution proposes a new key issue on algorithm negotiation for 256-bit algorithms.

# 4	Detailed proposals

Start of Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[aa]	3GPP TS 35.240: “Specification of the Snow 5G based 256-bits algorithm set: Specification of the 256-NEA4 encryption, the 256-NIA4 integrity, and the 256-NCA4 authenticated encryption algorithm for 5G; Document 1: Algorithm Specification”.

[bb]	3GPP TS 35.241: “Specification of the Snow 5G based 256-bits algorithm set: Specification of the 256-NEA4 encryption, the 256-NIA4 integrity, and the 256-NCA4 authenticated encryption algorithm for 5G; Document 2: Implementation Test Data”.

[cc]	3GPP TS 35.242: “Specification of the Snow 5G based 256-bits algorithm set: Specification of the 256-NEA4 encryption, the 256-NIA4 integrity, and the 256-NCA4 authenticated encryption algorithm for 5G; Document 3: Design Conformance Test Data”.

[dd]	3GPP TS 35.243: “Specification of the AES based 256-bits algorithm set: Specification of the 256-NEA5 encryption, the 256-NIA5 integrity, and the 256-NCA5 authenticated encryption algorithm for 5G; Document 1: Algorithm Specification”.

[ee]	3GPP TS 35.244: “Specification of the AES based 256-bits algorithm set: Specification of the 256-NEA5 encryption, the 256-NIA5 integrity, and the 256-NCA5 authenticated encryption algorithm for 5G; Document 2: Implementation Test Data”.

[ff]	3GPP TS 35.245: “Specification of the AES based 256-bits algorithm set: Specification of the 256-NEA5 encryption, the 256-NIA5 integrity, and the 256-NCA5 authenticated encryption algorithm for 5G; Document 3: Design Conformance Test Data”.

[gg]	3GPP TS 35.246: “Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: Algorithm Specification”.

[hh]	3GPP TS 35.247: “Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 2: Implementation Test Data”.

[ii]	3GPP TS 35.248: “Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 3: Design Conformance Test Data”.

Next Change

## 5.X	Key issue #X: Algorithm negotiation for 256-bit algorithms

### 5.X.1	Key issue details

Currently, 5G system supports the use of 128-bit algorithms for encryption and integrity protection in AS and NAS layer as described in clause 5.11 and 6.7 in TS 33.501 [3].

Additionally, specifications for 256-bit algorithms have been completed for 256-bit Snow 5G (TS 35.240 [aa], TS 35.241 [bb], TS 35.242 [cc]), 256-bit AES (TS 35.243 [dd], TS 35.244 [ee], TS 35.245 [ff]), and 256-bit ZUC (TS 35.246 [gg], TS 35.247 [hh], TS 35.248 [ii]). It's important to note that the authenticated encryption protection algorithm for these algorithms has also been introduced.

The key issue is how to negotiate these 256-bit algorithms in the 5G System. When comparing the negotiation with 128-bit algorithms, several issues need to be addressed, particularly in the context of supporting authenticated encryption protection algorithm:

- How to indicate UE support of 256-bit algorithms (including encryption, integrity protection, and authenticated encryption algorithm). For example, as specified in clause 9.11.3.54 of TS 24.501[4], the UE security capability IE has spare space, in which the 256 bits algorithm identifier(s) can be added

- Whether enhancements are needed on how to select the 256-bit algorithm on the network side.

- How to indicate the selected 256-bit algorithm.

- How to handle the case if the RRC selects an authenticated encryption algorithm, but UP protection only activates encryption or integrity protection.

- According to TS 33.501 [3], 128-bit integrity/encryption algorithms used for NAS/AS protection are negotiated via NAS/AS SMC message, which is only integrity protected. If the AE is selected, the AE may encrypt the whole NAS/AS SMC message. How to negotiate NAS/AS algorithms using authenticated encryption algorithm in 5GS is not clear.

### 5.X.2	Security threats

N/A.

### 5.X.3	Potential security requirements

The 5GS shall support the means to negotiate the new 256-bit algorithms (including encryption, integrity protection, and authenticated encryption algorithm) in AS and NAS layers.

End of Change