3GPP TSG-SA3 Meeting #115AdHoc-e	draft_S3-241218-r87

- Electronic meeting, online, 15 - 19 April 2024					          (revision of S3-yyxxxx)

- Source:	Huawei, HiSilicon, NOKIA, Nokia Shanghai Bell, Ericsson, Samsung?

- Title:	EEC provided information verification

- Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

Approve the changes in clause 4 for inclusion in the document: draftCR to TR 33.749

# 2	References

# 3	Rationale

Since no conclusion of EEC provided information verification was reached in R18, it was agreed in the Edge SID that this topic will be studied again in R19. The contribution proposes to shift the KI of EEC provided information verification in TR 33.739 to TR 33.749.

# 4	Detailed proposal

It is suggested to approve the following changes.

*************** Start of the 1st change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x1]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[x2]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

*************** End of the 1st change ****************

*************** Start of the 2nd change ****************

# 5	Key issues

### 5.Y	Key issue #Y: EEC provided information verification

#### 5.Y.1	Key issue details

Clause 8.6.5 of 3GPP TS 23.558 [x1] defines UE identifier API which is used by an EAS or EEC to obtain the identifier of the UE if the EAS or EEC does not have it (e.g. has not already cached). This identifier, called UE ID (could be the GPSI or the EEL-generated Edge UE ID, defined in clause 7.2.6 of 3GPP TS 23.558 [x1]), is used by the EAS to invoke capability APIs specific to UEs over EDGE-3 and/or EDGE-7 depending on the UE ID type.

The EES uses user information (e.g. IP address) received in the UE Identifier API invocation and obtains the UE identifier by interacting with NEF as specified in clause 4.15.10 of 3GPP TS 23.502 [x2]. The EES may utilize the Nnef_UEId_Get service (clause 4.15.10 of 3GPP TS 23.502 [x2]) providing the user information provided by the EEC.

Since the user information may be used to determine the UE ID, it is needed to ensure that the user information is trusted, and the EEC is authorized to use this user information as a parameter in the API invocation.

This key issue is to study the security and privacy aspects of usage of user information provided by the EEC in the UE ID API invocation.

#### 5.Y.2	Threats

If user information provided by the EEC is not verified and the EEC is not authorized to use this information, a malicious or compromised EEC or a malicious API consumer can try to execute IP address spoofing attacks to learn identifiers of other UEs, including those they are not authorized to access.

The usage of UE Identifier API invocation by the application server without any authorization by the UE may compromise the privacy of the UE.

Editor’s Note: Further security and privacy threats are FFS.

#### 5.Y.3	Potential security requirements

5G system should support a mechanism to verify the user information provided by EEC.

Editor’s Note: it is FFS on the requirement on the user privacy issue of the userage information and its usage.

Editor’s Note: It is FFS on whether the authorization of the usage of the user information by application servers (e.g., EAS) authorization is supported by exsisting mechanisms or not.

Editor’s Note: it is FFS the requirement aboutabout the authorization of the application servers (e.g. EAS) by the 5G system for the usage of the user information (i.e. IP address) on Nnef_UEID API.

*************** End of the 2nd change ****************