3GPP TSG-SA3 Meeting #115AdHoc-e	draft_S3-241404-r1

Electronic meeting, online, 15 - 19 April 2024

- Source:	Samsung

- Title:	Solution for SUPI privacy protection in PLMN hosted NPN scenario

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

It is proposed to approve this solution for addressing key issue#3.

# 2	References

NA

# 3	Rationale

It is proposed to approve this solution to address key issue#3.

# 4	Detailed proposal

*** Start 1st Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[xx]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

*** Start 2nd Change ***

## 6.Y	Solution #Y: SUPI privacy protection in PLMN hosted NPN scenario

### 6.Y.1	Introduction

This solution addresses the security requirement of key issue#3. This solution is applicable when the AMF is in the customer premises.

### 6.Y.2	Solution details

The UDM determines the AMF/SEAF is in the customer premises based on the SN name and/or NF ID received and generates and assign a temporary SUPI (SUPITemp) for the UE. Further the SUPITemp is used as the identity of the UE in the customer premises.

Figure 6.Y.2-1: Authentication and authorization procedure

- The UE sends the Registration Request NAS message to the SEAF/AMF in the serving network, containing either a SUCI (generated using the SUPI) or a 5G Globally Unique Temporary UE Identifier (5G-GUTI).

- On receiving the registration request, the SEAF/AMF invokes primary authentication by sending authentication request (i.e., Nausf_UEAuthentication_Authenticate request) to the AUSF in the home network containing the received SUCI and its serving network name (SNN).

- The AUSF verifies the SNN in the authentication request to check if it is the same as the expected serving name. If the verification is successful, it sends an authentication data request i.e., Nudm_UEAuthentication_Get request to the UDM, including the received SUCI and SN ID.

- On receiving the Nudm_UEAuthentication_Get Request, the UDM invokes SIDF if a SUCI is received. The SIDF de-conceals SUPI. The UDM/ARPF chooses the authentication method.

- The UDM provides the provides the temporary identifier indication to the AUSF in Nudm_UEAuthentication_Get Response message along with the other necessary parameters as specified in 6.1.3 of TS 33.501[xx]

6-7. The AUSF sends the authentication challenge message to the SEAF/AMF in a Nausf_UEAuthentication_Authenticate Response message including the freshness parameter along with other necessary parameters. The SEAF/AMF transparently forward the Challenge message to the UE in a NAS message Authentication Request message.

- The UE calculates the SUPITemp.  The SUPITemp derivation includes inputs to the KDF as: SUPI, SN ID, received Freshness parameter, key KAUSF. The UE uses the derived SUPITemp for further procedure, like derivation of KAMF.

Editor's Note: This solution has UE impact, how to avoid UE impact is FFS.

9-12. The procedure follows as specified in 6.1.3 of TS 33.501 [xx]. The AUSF derives SUPITemp in the same way as the UE in step 8 and stores the mapping between SUPI and SUPITemp. Instead of SUPI, AUSF provides the SUPITemp to the AMF/SEAF in the customer premises as subscription identifier of the UE. The AMF uses SUPITemp for further procedure.

13. Further, the message exchange follows procedure as specified in 6.1.3 of TS 33.501 [xx].

14. The AUSF shares the SUPITemp as part of Nudm_Authentication_result confirmation request along with the other necessary existing parameters to the UDM as specified in clause 6.1.4.1a of TS 33.501 [xx].

15. The UDM stores the mapping between SUPI and the SUPITemp as part of authentication results along with other parameters like authentication result, timestamp, and the serving network name.

16. The UDM replies to AUSF with a Nudm_UEAuthentication_ResultConfirmation Response.

The SUPITemp is used further as UEs subscriber identifier as long as the UE is served by the NF in the same customer premises. In other words, a new SUPITemp is assigned when UE is served by the NF in a different customer premises.

### 6.Y.3	Evaluation

TBD

*** End Change ***