3GPP TSG-SA3 Meeting #115AdHoc-e	S3-241419-r1

Electronic meeting, online, 15 - 19 April 2024

- Source:	Samsung

- Title:	New solution on AUN3 device mobility solution with Nonce

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This pCR proposes a new solution on KI#2 in TR 33.702.

# 2	References

NA

# 3	Rationale

This contribution proposes a new solution on KI#2.

# 4	Detailed proposal

*************** Start of 1st Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 33.887: " Study on security aspects for support for 5G Wireless and Wireline Convergence (5WWC) phase 2”

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

[4]	3GPP TS 23.502: "Procedures for the 5G System"

[xx]	RFC 5448: "Improved extensible authentication protocol method for 3rd generation authentication and key agreement (EAP-AKA')"

*************** Start of 2nd Change ****************

## 6.X	Solution #X: AUN3 device mobility solution with Nonce

### 6.X.1	Introduction

This solution addresses key issue #2: Security aspect of AUN3 device connecting to a new 5G-RG within the same W-AGF.

In this solution, full authentication may be skipped if AUN3 device is authenticated by a security context which is obtained from a previous full authentication.

### 6.X.2	Solution details

6.X.2.1	AUN3 device not supporting 5G key hierarchy

6.X.2.1.1	Procedure

Figure 6.X.2.1.1-1 5G-RG change procedure for AUN3 device not supporting 5G key hierarchy

- AUN3 device is connected to 5G-RG#1 as specified in clause 7B.7.2 in TS 33.501 [3] with the following changes:

  - AUN3 device generates a NonceAUN3 and sends it in EAP-ID-Response. It indicates that AUN3 device can derive PMK as specified in clause 6.X.2.1.2 of this document.

  - AMF, which knows the AUN3 device's capability, generates NonceAMF and 5G-GUTI after the primary authentication and AMF sends NonceAMF and 5G-GUTI to AUN3 device.

  - After the primary authentication, AMF and AUN3 device store MSK, 5G-GUTI and NonceAMF in the security context.

- AUN3 device decides to move from 5G-RG#1 to 5G-RG#2, so 802.11 connection is established between AUN3 device and 5G-RG#2.

- 5G-RG#2 initiates the EAP authentication procedure by sending an EAP request/Identity to the AUN3 device in a layer 2 frame.

- The AUN3 generates a new NonceAUN3 and MACAUN3, before sending back an EAP response/Identity including 5G-GUTI, NonceAUN3, and MACAUN3. MACAUN3 is generated using MSK and NonceAMF received in step 1. MACAUN3 may be the part of the output (e.g., most significant 128-bits of the output).

- 5G-RG#2 sends a NAS Registration Request message to the AMF, including 5G-GUTI, AUN3 device indicator, NonceAUN3, and MACAUN3.

- If the AMF is different from the old AMF which allocated the 5G-GUTI in step 1, the new AMF retrieves SUPI and security context from the old AMF. AMF verifies the MACAUN3 using MSK and NonceAMF stored in the security context. If the verification is successful and AMF decides to skip a full authentication based on an operator's policy, the full authentication is skipped. AMF generates a new NonceAMF and 5G-GUTI. AMF also computes MACAMF using existing MSK and NonceAUN3 received in step 5. MACAMF may be the part of the output (e.g., most significant 128-bits of the output). AMF stores MSK, 5G-GUTI, and newly generated NonceAMF in its security context.

- AMF derives PMK as specified in clause 6.X.2.1.2 using existing MSK, NonceAUN3 received in step 5, NonceAMF generated in step 6.

- AMF sends EAP-Success and PMK to the 5G-RG#2 in N1 message.

- The 5G-RG#2 sends to the AUN3 device the EAP-Success message including 5G-GUTI, NonceAMF, and MACAMF.

- The AUN3 device verifies the MACAMF using NonceAUN3 generated in step 4 and existing MSK. If the verification is successful, AUN3 derives PMK as specified in clause 6.X.2.1.2 using existing MSK, NonceAUN3 generated in step 4, and NonceAMF received in step 9. The AUN3 device stores 5G-GUTI, the newly received NonceAMF, and MSK in its security context.

- If the layer 2 connection is over WLAN, the AUN3 device and the 5G-RG#2 use the first 256-bit of the PMK for WLAN keys derivation.

- The AUN3 device and the 5G-RG#2 performs four-way handshaking to establish WLAN secure connection.

6.X.2.1.2	PMK key derivation

Figure 6.X.2.1.2-1 Key hierarchy for AUN3 device not supporting 5G key hierarchy

PMK can be derived in AMF/AUN3 device as follows:

PMK = PRF'(most significant 256 bits of MSK, NonceAUN3|NonceAMF)[640..1151], as specified in RFC 5448 [xx].

6.X.2.2	AUN3 device supporting 5G key hierarchy

6.X.2.2.1	Procedure

Figure 6.X.2.2.1-1 5G-RG change procedure for AUN3 device supporting 5G key hierarchy

- AUN3 device is connected to 5G-RG#1 as specified in clause 7B.7.3 in TS 33.501 [3] with the following changes:

  - AUN3 device generates NonceAUN3 and sends it in EAP-Res/Identity. It indicates that AUN3 device can perform horizontal key derivation on KAMF as specified in clause 6.X.2.2.2 of this document.

  - AMF, which knows the AUN3 device's capability, generates NonceAMF and 5G-GUTI after the primary authentication and AMF sends NonceAMF and 5G-GUTI to AUN3 device.

  - After the primary authentication, AMF and AUN3 device store KAMF, 5G-GUTI and NonceAMF in the security context.

- AUN3 device decides to move from 5G-RG#1 to 5G-RG#2, so 802.11 connection is established between AUN3 device and 5G-RG#2.

- 5G-RG#2 initiates the EAP authentication procedure by sending an EAP request/Identity to the AUN3 device in a layer 2 frame.

- The AUN3 generates a NonceAUN3 and MACAUN3, before sending back an EAP response/Identity including 5G-GUTI, NonceAUN3, and MACAUN3. MACAUN3 is generated using KAMF and NonceAMF received in step 1. MACAUN3 may be the part of the output (e.g., most significant 128-bits of the output).

- 5G-RG#2 sends a NAS Registration Request message to the AMF, including 5G-GUTI, AUN3 device indicator, NonceAUN3, and MACAUN3.

- If the AMF is different from the old AMF which allocated the 5G-GUTI in step 1, the new AMF retrieves SUPI and security context from the old AMF. AMF verifies the MACAUN3 using KAMF and NonceAMF stored in the security context. If the verification is successful and AMF decides to skip a full authentication based on an operator's policy, the full authentication is skipped. AMF generates a new NonceAMF and 5G-GUTI. AMF also computes MACAMF using KAMF and NonceAUN3 received in step 5. MACAMF may be the part of the output (e.g., most significant 128-bits of the output).

- AMF performs horizontal key derivation on KAMF as specified in clause 6.X.2.2.2 using existing KAMF, NonceAUN3 received in step 5, NonceAMF generated in step 6. AMF stores newly derived KAMF, 5G-GUTI, and newly generated NonceAMF in its security context. AMF derives KWAGF from the newly derived KAMF as specified in Annex A.9 in TS 33.501 [3].

- AMF sends EAP-Success and PMK to the 5G-RG#2 in N1 message.

- The 5G-RG#2 sends to the AUN3 device the EAP-Success message including 5G-GUTI, NonceAMF, and MACAMF.

- The AUN3 device verifies the MACAMF using NonceAUN3 generated in step 4 and existing KAMF. If the verification is successful, AUN3 performs horizontal key derivation on KAMF as specified in clause 6.X.2.2.2 using existing KAMF, NonceAUN3 generated in step 4, and NonceAMF received in step 9. The AUN3 device stores 5G-GUTI, the newly received NonceAMF, and newly derived KAMF in its security context. AUN3 device derives KWAGF from the newly derived KAMF as specified in Annex A.9 in TS 33.501 [3].

- If the layer 2 connection is over WLAN, the AUN3 device and the 5G-RG#2 use KWAGF as the PMK, from which the WLAN keys are derived.

- The AUN3 device and the 5G-RG#2 performs four-way handshaking to establish WLAN secure connection.

6.X.2.2.2	Horizontal key derivation on KAMF

Horizontal key derivation on KAMF is performed similar to Annex A.13 in TS 33.501 [3] with some changes:

Derivation of KAMF' from KAMF during mobility in AUN3 device supporting 5G key hierarchy uses the following input parameter:

- FC = 0xXX

- P0 = NonceAUN3

- L0 = length of NonceAUN3

- P1 = NonceAMF

- L1 = length of NonceAMF

The input key KEY is KAMF.

NOTE:	FC value to be determined during normative phase.

### 6.X.3	Evaluation

This solution has impact on AMF.

Editor's Note:	Further evaluation on impact to entities is FFS.

Editor's Note:	Further evaluation is FFS.