3GPP TSG-SA3 Meeting #115e ad-hoc	S3-241431r1

Electronic meeting, online, 15 - 19 April 2024

- Source:	Qualcomm Incorporated

- Title:	Solution to KI#4 using FT

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes a solution to KI#4 using FT.

# 2	References

[1]	S3-240627 Draft TR 33.702 v0.1.0

[2]	TS 33.501

# 3	Rationale

As described in Annex S of TS 33.501 [2], for NSWO, the MSK is passed done to the WLAN AP which means that FT can be used directly for mobility without any signalling changes. It is proposed to capture this in the TR [1] as a solution to KI#4 on NSWO. An editorial in KI#4 is also proposed.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [1].

**** START OF CHANGES ****

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 33.887: "Study on security aspects for support for 5G Wireless and Wireline Convergence (5WWC) phase 2".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[4]	3GPP TS 23.502: "Procedures for the 5G System".

[aa]	IEEE Std 802.11™-2020 Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

**** NEXT CHANGE ****

- 5.4.1	Key issue details

When UE moves from WLAN AP1 to WLAN AP2, where both WLAN APs are nearby or overlapping, the UE connectivity can break while connecting to the new WLAN AP. Additionally, UE also goes through another full authentication procedure at the NSWOF as defined in annex S of TS 33.501 [3], even though both WLAN APs connect to the same NSWOF.

A new full authentication may lead to additional signalling and may cause latency in the UE connection. This is inefficient and disrupts the user experience. If we skip the full authentication, it provides connection time optimisation, but then the security aspect of the UE to target WLAN should be studied.

**** NEXT CHANGE ****

- 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

| Solutions | KI#1 | KI#2 | KI#3 | KI#4 |
| --- | --- | --- | --- | --- |
| Solution #Y: Using FT for NSWO |  |  |  | x |

Editor’s Note: Each solution should be mapped here.

**** NEXT CHANGE ****

## 6.Y	Solution #Y: Using FT for NSWO

### 6.Y.1	Introduction

This solution addresses key issue #4: Security aspect of UE connecting to a new WLAN AP connected via the same NSWOF.

### 6.Y.2	Solution details

This solution is similar to solution #3 (see 6.3 of the present document) except that there is no need to derive a new key (KFT in solution #3) as MSK provided to the WLAN Access Network (AN) (see Annex S of TS 33.501 [3]). MPMK is derived directly from MSK as described in 802.11 [aa]. Hence no standards changes are needed in this case to deploy FT as there is no need to derive the MPMK.  Put another way when using FT, MSK = PMK || MPMK

To realise this solution, the WLAN AN that the UE is connecting needs to support FT and hence contains a R0HK. The R0KH receives the MSK and takes the MPMK to generate the FT root key. The R0KH passes the PMK to R1KH (i.e., AP) so the R1KH can complete the 4-way handshake with the UE.

### 6.Y.3	Evaluation

TBD

**** END OF CHANGES ****